# Hearing Safety

Instructor Guide

## Safety Talk Overview:

What & Why? Safety Talks are a method to refresh an employee’s knowledge and skills, maintain their interest in safety and illustrate the organization’s commitment to creating a healthy & safe work environment.

Who & When? Generally, these Safety Talk meetings are led by a supervisor, member of the JHSC, or Worker H&S Representative and should be mandatory for all crew members to attend. The content should be focused on a single topic. Safety Talks can be performed on a weekly basis or before the start of a new scope of work and should be about 15 minutes in duration.

go2HR developed this safety talk template for employers to customize and use for any topic in their organization. The Participant Handout should include information that you hope the worker will retain from the safety talk content and should be handed out during the safety talk for workers to reference later. Feel free to use it at your discretion.

Hearing Safety Overview

Exposure to excessive noise, if not eliminated or controlled properly, can cause permanent hearing loss. Examples of occupations working with hazardous noise levels at ski areas include lift operators/maintenance, snow makers, truck drivers, mobile equipment operators, carpenters, maintenance personnel, and individuals working in a shop. Be aware that noise-induced hearing loss is 100% preventable. However, once hearing is lost, it is permanent and irreversible. Exposure to noise can have adverse effects such as reducing productivity; creating stress; interfering with communications; and making it difficult to hear equipment or warning signals.

Safety Talk Outline (using the handout below):

- Preventative Measures

- Discussion Questions

Additional Resources:

- go2HR Ski Areas

-WorkSafeBC Noise

## Topic Overview:

## Safety Talk Facilitator Guidance

Introduce the topic and why it’s important

Discuss the associated hazards and likely incidents

Tour the work zone identifying hazardous areas

Distribute the Participant Handout

Use the info above to guide the discussion

Document the talk using the Safety Talk Record

Explain what controls are used to minimize the risks

Remind employees about applicable PPE usage

Emphasize the importance of safe work procedures

Ask questions to generate group discussion

Answer any questions or concerns they might have

Set a good example by working safely at all time

# hearing safety

Safety Talk Record

| ### Discussion Leader: | ### Date: |
| --- | --- |
| ### Department: | ### Time: |

### Attendees (Please print your name and sign beside it. If you are a contractor, also include your company name):

### Near miss/incidents and investigations reviewed:  ☐ None this month

### Any questions or concerns from workers?

| ### Action needed: | ### Person responsible: | ### Due date: | ### Completed date: |
| --- | --- | --- | --- |

| ### Supervisor/Manager print name & sign: | ### Date: |
| --- | --- |

### Reviewed by:

# hearing safety

Exposure to excessive noise, if not eliminated or controlled properly, can cause permanent hearing loss. Examples of occupations working with hazardous noise levels at ski areas include lift operators/maintenance, snow makers, truck drivers, mobile equipment operators, carpenters, maintenance personnel, and individuals working in a shop. Be aware that noise-induced hearing loss is 100% preventable. However, once hearing is lost, it is permanent and irreversible. Exposure to noise can have adverse effects such as reducing productivity; creating stress; interfering with communications; and making it difficult to hear equipment or warning signals.

Preventative Measures

Noise Level Assessment

-Rule of thumb: If conversation requires shouting, noise levels may exceed 85 dBA, necessitating protection

Selection of Hearing Protection Devices (HPD)

Select HPDs based on:

-Noise exposure

-Communication demands

-Hearing ability

-Use of personal protective devices

-Temperature and climate

-Physical characteristics of the job or worker

Proper Insertion of Foam Earplugs

1. Roll and compress the entire earplug into a thin cylinder using clean hands

2. Pull back and up on the outer part of the ear to facilitate insertion

3. Insert the earplug into the ear canal, holding it in place until it expands and effectively blocks noise

Noise Management Strategies

-Schedule noisy machines during times when fewer employees are present

-Apply vibration-absorbing materials onto walls to minimize noise transmission

Understanding Hearing Protection Options

-Ear Plugs: Inserted to block the ear canal

-Semi-insert Plugs: Two ear plugs held over the ends of the ear canal by a rigid headband

-Ear Muffs: Sound diminishing cushions that fit around the ear and are held in place with a headband

Recognizing When Hearing Protection is Necessary

Wear hearing protection when:

-Shouting is required to be heard

-Ears are roaring or ringing at the end of the workday

-Speech or music sounds muffled after work but clear in the morning

Mandatory Participation in Annual Hearing Tests

-Participate in annual hearing tests as required by WorkSafeBC

Discussion

-Discuss some areas in the workplace where hearing protection should be worn

-Which type of hearing protection is most suitable for each of the areas you just discussed?

Participant Handout

### For more information on this topic or if you have questions, contact: