|  | Industry/Sector: | Ski Hill and Areas | Department/Job Position: | Lift Operation |
| --- | --- | --- | --- | --- |
|  | Common Tasks and Duties: | - Assisting guests with loading/unloading chairlifts, gondolas, and surface lifts (e.g. conveyer carpets, rope tow, T-bars etc.) - Monitoring lift operations and safety systems (e.g. control panels, CCTV, communication, safety devices etc.) - Performing daily pre-start inspections (e.g. brakes, cables, emergency stops, towers etc.) - Clearing snow and ice from lifts, loading/unloading areas, ramps, walkways and stairs - Managing lift lineups and crowd management - Responding to equipment faults, alarms, or emergency stops - Assisting patrol and maintenance in lift rescue procedures - Providing customer service, instruction and directions to guests | Reasonably Foreseeable or Common Hazards: | - Slips, trips and falls, including tree wells, snow instability, embankments and other hill hazards - Moving/rotating parts causing struck by/against and pinch point hazards (e.g. machinery/equipment) - Collisions with objects, guests, workers, equipment - Electrical, noise and vibration hazards - Working from heights - Falling objects (e.g. ice, snow, guest items) - Musculoskeletal injuries (MSI) and ergonomics - Harassment and violence, working alone - Severe weather and cold stress - Fatigue and psychological hazards |

Employers are to review this HIRA document and customize to their location, job tasks and site-specific hazards. To edit a row, simply use Microsoft word to edit (e.g. edit text, change risk ratings, delete a row, or add a new row). When determining risk follow the Guidance Document - Assessing Risk for HIRA Tools. When identifying controls, be sure to use the Hierarchy of Controls to control risk through elimination, substitution, engineering controls, administrative controls and Personal Protective Equipment (PPE). Once complete, this should be reviewed by the JHSC or a Worker H&S Rep, and reviewed annually thereafter.

|  | Full Name | Job Position | Signature | Date |
| --- | --- | --- | --- | --- |
| Completed By: |  |  |  |  |
| Reviewed By: |  |  |  |  |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Travel to work location (e.g. walk and/or ski/board to lifts) | - Slips, trips and falls - Collisions with objects, guests, workers, or equipment - Tree wells, snowpack instability, embankments and hill hazards - Working alone and in isolation - MSI (strains/sprains) risks - Severe weather and cold stress - Harassment and violence | 2 Possible | 3 Serious | 6 Med. Risk | - Recruitment, training, skills assessment, and supervision should confirm competencies/abilities to safely travel on hills - Working Alone Policy with established check-in procedures - Designated routes and clear communication and briefings - Mark hazards, and use buddy-system and radios/GPS - Ensure functioning of radios, Sat. comms/inReach devices - Proper equipment and materials, including PPE (e.g. safety headgear, headlamp, eye protection and reflective materials) - See HIRA for general control measures for MSI risks, cold stress and harassment/violence if working with the public | 1 Unlikely | 3 Serious | 3 Low Risk | Briefings to provide overview on daily hazards, expected weather, fit for duty, planned and alternate routes, safe work practices, PPE, and rescue/emergency procedures |
| Pre-operational checks and setting up line control | - Struck by/against (lift, equipment) - Falling ice/snow or other objects - Falls from step stools or ladders - Hand or other injuries when using tools (e.g. drills) and equipment - Exposure to energized equipment during inspection - MSI risks from moving barriers, gates, line control and equipment - Severe weather and cold stress | 3 Likely | 3 Serious | 9 High Risk | - Only qualified and trained personnel to perform pre-operational checks, electrical work and working from heights - Provide workers with instruction and training - De-energize equipment before performing maintenance - Lockout/tagout (LOTO) procedures with individual locks/tags - Follow safe work procedures and wear required PPE - Use tools, panels and equipment rated for outdoor use - Avoid working at heights in poor weather conditions - Set up work areas to prevent tools/materials and ice/snow from falling and use work delineation (e.g. barriers, cones) | 2 Possible | 3 Serious | 6 Med. Risk | Safe ladder use includes pre-use inspection of ladders and equipment, ensuring CSA or ANSI-certified, maintaining 3-points of contact, training, ensuring proper set-up and using work delineation and spotters, see WorkSafeBC Ladder Safety |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Operating control panels and equipment | - Electrical hazards from faulty or damaged equipment - Mechanical hazards, inadvertent energization or start-up - Human error or communication failures causing injuries or equipment malfunction or failure - Noise and vibration hazards - MSI risks from operating panels and workstation set-up - Severe weather and cold stress | 2 Possible | 3 Serious | 6 Med. Risk | - Only qualified and trained staff to operate control panels and equipment including emergency stop and restart protocols - Standardized labelling of controls and established checklists - Follow manufacturer’s instruction for start-up and safe use - Ensure functioning of radios, Sat. comms/inReach devices - Improve visibility through adequate lighting, CCTV, convex mirrors and spotters - Regular equipment inspection and maintenance, including audible and visual alarms - De-energize equipment before performing maintenance - Lockout/tagout (LOTO) procedures with individual locks/tags - Maintain situational awareness, avoid distractions and prohibit cellphone use | 1 Unlikely | 3 Serious | 3 Low Risk | Access to operating control panels and equipment should be limited to authorized personnel only |
| Clearing snow/ice from stations and maintaining ramp | - Slips, trips and falls - Falling ice/snow or other objects - Falls from step stools or ladders - Severe weather and cold stress - Significant MSI risks due to heavy and rigorous movement associated with snow clearing and ramp maintenance | 3 Likely | 3 Serious | 9 High Risk | - Regular de-icing and snow removal to minimize risk of excessive snow and ice build-up or equipment failure - Training on proper shovelling and snow removal techniques - Different sized shovels and ergonomic/extendable tools - De-icing roof of huts and structures, regular snow clearing - Set up work areas to prevent tools/materials and ice/snow from falling and use work delineation (e.g. barriers, cones) - Proper footwear and traction aids (e.g. cleats, Yaktrax, mats) - Supervision to monitor correct techniques and controls - See HIRA for general control measures for MSI risks, cold stress and harassment/violence if working with the public | 2 Possible | 3 Serious | 6 Med. Risk | Use snowblowers only when trained and when safe to operate without creating risks to other workers or guests in the area |
| Assisting guests with loading/unloading including assisting with dropped items or equipment | - Slips, trips and falls - Collisions with or between guests and workers - Struck by/against from ski lift, guests, or guest ski equipment - Caught in or pinch point hazards - Severe weather and cold stress - MSI risks from assisting guests and flipping/clearing seats - Harassment and violence | 3 Likely | 3 Serious | 9 High Risk | - Training on safe loading/unloading techniques along with hazard awareness of caught in and pinch point hazards - Maintain clear loading zones and limit materials in area - Control speed and slow speeds during loading process - Easily accessible and no obstructions for emergency stop - Clear guest signage, communication and announcements - Convex mirrors and lighting to increase visibility - Crowd management and maintain spacing of guests - Maintain situational awareness and lines of sight - Regular de-icing and snow removal to minimize risk of stuck chairs or equipment failure or malfunction - Proper footwear and traction aids (e.g. cleats, Yaktrax, mats) - Supervision to monitor correct techniques and controls | 2 Possible | 3 Serious | 6 Med. Risk | Increased risk and attention needed for monitoring children, beginners, ski limos, seat skiers, and other guests requiring additional assistance During peak times, ensure adequate staffing for crowd management and guest loading/unloading |
| Musculoskeletal injuries (MSI) and ergonomics | - Repetitive motions when scrap-ing ice, shovelling snow, or using tools to clear lifts and equipment - Awkward postures when bending, or reaching to assist guests, adjust bars, or clear snow - Overexertion when lifting or maneuvering heavy or awkward items such as shovelling snow, lifting gates, barriers, equipment - Pushing or pulling when repositioning gates, moving conveyor mats, or handling tools - Improper techniques assisting guests or handling supplies | 3 Likely | 3 Serious | 9 High Risk | - Training on MSI prevention, ergonomics, safe lifting, shovelling, positioning, and guest-assist techniques - Reminders during pre-shift briefings on MSI prevention - Warm-up, stretch and take breaks including micro-breaks - Rotate tasks (e.g. shovelling) throughout the shift - Use proper lifting techniques or mechanical aids - Different sized shovels and ergonomic/extendable tools - Break heavy loads (e.g. sand/salt bags) into smaller loads - Use team lifts for awkward items (lifting gates, equipment) - Use equipment with wheels, sliders, or handles, if possible - Push carts instead of pulling to reduce risk of MSIs - Report and replace worn or broken equipment - Store heavier items at waist level to avoid unnecessary lifting - Encourage early reporting of discomfort or MSI symptoms | 2 Possible | 3 Serious | 6 Med. Risk | See resources for employers from WorkSafeBC and go2HR on MSI prevention Encourage maintenance of physical fitness during off-season and throughout season |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Line control and crowd management | - Slips, trips and falls - Collisions and struck by/against incidents for guests and ski equipment - Congestion and crowd control - Aggressive guests, potential for harassment and violence - Severe weather and cold stress | 3 Likely | 2 Moderate | 6 Med. Risk | - Proper set-up, spacing and shoots for line control and barriers, based on anticipated attendance and use - Physical barriers (e.g. ropes, fencing, cones, delineators), ensuring ability for emergency access if needed - Regular grooming and clearing of lines - Applying sand, grit and salt and traction aids, as applicable - Clear signage to identify lineup etiquette and structure - Visible staff with polite and direct enforcement of rules and line management to guests with extra staff at peak times - Maintain situational awareness and crowd monitoring | 2 Possible | 2 Moderate | 4 Med. Risk | Communication of lift wait times to guests so they can plan runs and activities accordingly |
| Severe weather and cold stress | - Cold stress injuries such as frostbite and hypothermia - Impaired decision-making and slower reaction times - Slips, trips, and falls from snow and ice accumulation on surfaces - Reduced visibility from blowing snow, fog, or whiteout conditions - Equipment malfunctions or failures caused by ice buildup or frozen components - Sun exposure (UV risks) can be increased with reflection from snow and at higher altitudes | 3 Likely | 3 Serious | 9 High Risk | - Training on cold stress prevention and reminders during briefings, including early warning signs of cold stress - Ensure staff dress in layers and insulated, waterproof and wind-resistant clothing, gloves, headgear and boots - Equip workers with high-vis and reflective outerwear - Schedule regular breaks and rotations in heated shelters - Provide warm, non-caffeinated drinks in break areas and encourage staying hydrated - Perform buddy-checks to monitor alertness and cold stress - Encourage proper rest, nutrition, and hydration before shifts - Keep work areas, platforms, and paths clear of snow/ice and apply sand, salt, grit, or traction aids to walking surfaces - Conduct frequent inspections and de-icing of lift equipment - Report and replace worn or broken equipment - Wear UV-rated sunscreen and eye protection, as applicable | 2 Possible | 3 Serious | 6 Med. Risk | See resources for employers from WorkSafeBC and go2HR on cold stress prevention Suspend or modify lift operations when visibility and temperature fall below safe limits or other unsafe conditions as part of weather monitoring and emergency plans |
| Harassment and violence due to working with the public | - Verbal abuse during long lines, delays, or closures - Threatening or aggressive behaviour when enforcing rules - Physical altercations (pushing, grabbing, striking) - Sexual harassment (comments, gestures, unwanted contact) - Stress from repeated or escalating incidents - Working alone at stations or if travelling without immediate help | 2 Possible | 3 Serious | 6 Med. Risk | - Harassment and violence prevention program in accordance with WorkSafeBC requirements - Training and awareness on harassment, violence, conflict resolution, de-escalation, and recognizing warning signs - Clear rules and signage for use, line queuing and conduct - Reporting procedures for response and incident reports - Working alone protocols including regular check-ins when stationed remotely or travelling between lift stations - Devices for immediately summoning assistance (e.g. radios, apps, Sat. comms, inReach, GPS, panic button, whistle etc.) - Increased security or patrol presence during peak periods - Support resources such as post-incident debriefing, mental health supports, or employee assistance programs | 2 Possible | 2 Moderate | 4 Med. Risk | See resources for employers from WorkSafeBC and go2HR on workplace harassment and violence prevention Avoid work with increased risk when working alone. When possible, have staff work in pairs or groups, particularly in locations and with tasks that are increased risk |
| Responding to emergencies (e.g. medical, severe weather, power outages, equipment failure) | - Malfunction or failure of lift systems - Worker/guest medical emergency - Falls from heights - Guest panic, crowding - Severe weather and cold stress - Power loss affecting lighting, equipment, and safety systems - Increased risk of injuries due to rushing or working under stress - Disruption to business operations | 3 Likely | 3 Serious | 9 High Risk | - Only trained and qualified personnel to perform emergency and rescue operations with emergency equipment and PPE - Emergency plans in place with procedures for responding to reasonably foreseeable emergency events - Ensure first aid assessment and medical program in place - Scene assessment (e.g. unstable structures/snow, traffic, falls, electrical risks etc.) to confirm safe for response - Tabletop exercises and emergency drills to reinforce training - Escalate calls to service techs and emergency services - Provisions for debriefing and psychological safety and mental health supports for workers involved in or witnessed critical/traumatic incidents - Business continuity with crisis communication plans in place | 2 Possible | 3 Serious | 6 Med. Risk | Ensure first aid requirements are in place in accordance with WorkSafeBC (including drills), and system in place to ensure certifications and equipment (e.g. kits and AEDs) are up to date and maintained Ensure rescue team is trained, regular drills conducted, and rescue equipment (e.g. harness, rope) and kit are inspected and maintained |