|  | Industry/Sector: | Ski Hill and Areas | Department/Job Position: | Grooming |
| --- | --- | --- | --- | --- |
|  | Common Tasks and Duties: | - Operating snowcats and other mobile equipment on trails, terrain parks, remote locations, and in close proximity to lifts, towers, and overhead cables or electrical equipment - Coordinating with terrain park and lift operation staff, and snowmaking teams - Connecting and disconnecting from winch cables systems - Refueling and performing basic equipment checks - Working alone at night in low-visibility conditions - Communicating with dispatch, patrol, and lift operations | Reasonably Foreseeable or Common Hazards: | - Slips, trips and falls, including when climbing or dismounting from equipment - Loss of control or rollover of equipment on steep, icy, unstable snow or uneven terrain - Blind spots contributing to collisions - Breakdown in remote areas, working alone - Moving/rotating parts causing struck by/against and pinch point hazards (e.g. machinery/equipment) - Electrical, noise and vibration hazards - Musculoskeletal injuries (MSI) and ergonomics - Severe weather and cold stress - Limited visibility during nightwork - Fatigue and psychological hazards |

Employers are to review this HIRA document and customize to their location, job tasks and site-specific hazards. To edit a row, simply use Microsoft word to edit (e.g. edit text, change risk ratings, delete a row, or add a new row). When determining risk follow the Guidance Document - Assessing Risk for HIRA Tools. When identifying controls, be sure to use the Hierarchy of Controls to control risk through elimination, substitution, engineering controls, administrative controls and Personal Protective Equipment (PPE). Once complete, this should be reviewed by the JHSC or a Worker H&S Rep, and reviewed annually thereafter.

|  | Full Name | Job Position | Signature | Date |
| --- | --- | --- | --- | --- |
| Completed By: |  |  |  |  |
| Reviewed By: |  |  |  |  |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Operating grooming equipment and snowcats | - Slips, trips, and falls - Collisions with objects, guests, workers, or equipment - Struck by attachments (blades, rakes) or entanglement from winch cables - Limited operator visibility leading to “blind spots” - Overhead hazards/low clearance - Loss of control or rollover on steep, icy, or uneven terrain - Breakdown of grooming equipment or snowcat - Working alone and in isolation - Wildlife on trails or remote routes - MSI (strains/sprains) risks - Severe weather and cold stress - Psychological safety and fatigue | 3 Likely | 3 Serious | 9 High Risk | - Recruitment, training, skills assessment, and supervision should confirm competencies/abilities to operate equipment - Follow PPE requirements for safety headgear, eye protection, hearing protection and reflective materials - Operate within equipment manufacturer’s slope limitations - Maintain controlled speed appropriate to terrain and visibility - Conduct pre-use inspection of snowcat/grooming equipment - Ensure safety devices (e.g. alarms, lights, mirrors, Roll Over Protective Structure (ROPS) etc.) are in good condition - Load carrying and tow restrictions, ensure loads secured - Plan travel in advance and follow designated routes - Assess routes for hazards including overhead powerlines and lift cables and structures - Ensure operators and passengers wear seatbelts - Working Alone Policy with established check-in procedures - Maintain situational awareness, avoid distractions - Devices for immediately summoning assistance (e.g. radios, apps, Sat. comms, inReach, GPS, panic button, whistle etc.) - Carry emergency kit and gear (e.g. first aid kit, lighting, blanket, survival pack, tow rope, extra fuel etc.) | 2 Possible | 3 Serious | 6 Med. Risk | Briefings to provide overview on daily hazards, expected weather, fit for duty, planned and alternate routes, safe work practices, PPE, and rescue/emergency procedures Suspend or modify grooming operations when visibility and temperature fall below safe limits or other unsafe conditions as part of weather monitoring and emergency plans Ensure emergency procedures include contact with overhead powerlines, see BC Hydro and WorkSafeBC resources |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Loading/unloading, riding in, and working in close proximity to snowcats | - Slips, trips and falls, particularly when climbing/demounting - Cuts/pinch points from equipment - Struck against/by attachments (doors, tracks, blades, rakes) - Entanglement from winch cables - Limited operator visibility leading to “blind spots” - Noise and loud environments - Uneven (sloping/unstable) terrain affecting stability and clearance - Severe weather and cold stress | 3 Likely | 3 Serious | 9 High Risk | - De-ice and establish clear pathways and approach zones - Only qualified and trained workers permitted in work zone, keep clear of tracks and attachments - Provide training on potential blind spots and hand signals - Proper equipment and materials, including PPE (e.g. safety headgear, gloves, eye protection and high-vis clothing) - Ensure functioning of radios, Sat. comms/inReach devices - Pre-use inspections to ensure safety devices (e.g. lights, alarms etc.) are operational and ladders in good condition - Approach grooming equipment after it has stopped, and only after eye contact with the operator and signal confirmation - Maintain three points of contact while climbing/demounting - Face the machine and use designated step and handrails - Do not carry materials in your hand, use a backpack | 2 Possible | 3 Serious | 6 Med. Risk | Briefings to provide overview on daily hazards, expected weather, fit for duty, planned and alternate routes, safe work practices, PPE, and rescue/emergency procedures Ensure adequate lighting and visibility - if not possible, take additional precautions (e.g. spotters, closures etc.) |
| Clearing snow/ice from equipment and attachments (e.g. tiller, blades, tracks) | - Slips, trips and falls - Falling ice/snow or other objects - Severe weather and cold stress - Significant MSI risks due to heavy and rigorous movement associated with snow clearing activities | 3 Likely | 3 Serious | 9 High Risk | - Regular de-icing and snow removal to minimize risk of excessive snow and ice build-up or equipment failure - Training on proper shovelling and snow removal techniques - Different sized shovels and ergonomic/extendable tools - De-icing equipment and structures, regular snow clearing - Proper footwear and traction aids (e.g. cleats, Yaktrax, mats) - Supervision to monitor correct techniques and controls | 2 Possible | 3 Serious | 6 Med. Risk | Use snowblowers only when trained and when safe to operate without creating risks to other workers or guests in the area If risk of moving parts, implement controls as outlined below |
| Working around moving parts (e.g. tracks, winches, cables, tillers) | - Entanglement, pinch point or caught-in hazards - Struck-by hazards from moving tracks, attachments, broken cables, and mechanical components - Noise and vibration hazards - Severe weather and cold stress | 3 Likely | 3 Serious | 9 High Risk | - Train workers on recognition of pinch points, entanglement hazards, and safe approach distances - Use guarding and barriers around exposed moving parts - De-energize equipment before performing maintenance - Lockout/tagout (LOTO) procedures with individual locks/tags - Follow manufacturer’s instruction for safe use, pre-use inspection and maintenance - Avoid loose clothing, secure hair, jewellery, and ID badges - Maintain situational awareness, avoid distractions - Wear PPE (e.g. gloves, safety glasses, hearing protection) | 2 Possible | 3 Serious | 6 Med. Risk | If contracting work with increased risk, ensure contractor safety is managed through controls such as pre-qualification, confirmation of valid insurance (including WSBC clearance letter), hazard assessments, verification of qualifications and training, and safe work procedures and equipment |
| Using chemicals and hazardous products (e.g. oils, solvents, cleaners, fuels, lubricants) | - Exposure to chemical hazards (e.g. chemical burns, splashes to eyes, respiratory or skin irritation) - Fire hazards when refuelling - Unlabelled or illegible containers - Spills and environment concerns | 2 Possible | 3 Serious | 6 Med. Risk | - WHMIS training for staff using chemical products - Supplier and workplace labels in place and legible - Safety Data Sheets (SDS) referenced and available - Safe handling practices and PPE as required by label/SDS - Ensure no smoking and safe work procedures for refuelling - Fuel tanks to be properly stored with bonding/grounding and flame arrestors with workers trained in safe use - Ensure proper storage of chemicals and hazardous products | 1 Unlikely | 2 Moderate | 2 Low Risk | Ensure spill and environmental procedures in place for fuel or other chemical spills in accordance with SDS |
| Working with hydraulic and pressurized systems (e.g. brakes, hoses, jacks, cylinders, compressed air) | - Slips, trips and falls - High-pressure fluid release causing injection injuries or burns - Hose or fitting rupture, failure or malfunction releasing energy - Eye injuries from flying debris or materials - Crushing or pinching hazards - Contact with hydraulic fluids (see chemical hazards within HIRA) - Noise and vibration hazards | 3 Likely | 3 Serious | 9 High Risk | - Only qualified and trained personnel to work with hydraulic and pressurized systems and equipment - De-energize equipment before performing maintenance, and relieve system pressure before disconnecting or servicing - Use guarding, blocks, or mechanical restraints to prevent unintended movement of parts - Lockout/tagout (LOTO) procedures with individual locks/tags - Use manufacturer-specified tools, fittings, and rated hoses to prevent leaks and failures, do not substitute with other parts - Inspect hoses, fittings, and cylinders regularly for wear, cracks, or damage, and replace immediately - Wear PPE (e.g. gloves, safety glasses, hearing protection) | 2 Possible | 3 Serious | 6 Med. Risk | Post clear rules around working with compressed air and ensure gas regulators are in place Report and repair leaks immediately, do not operate equipment with known hydraulic defects |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Connecting and disconnecting winch cables | - Slips, trips and falls - Entanglement, pinch point or caught-in hazards - Struck-by hazards from moving tracks, attachments, broken cables, and equipment - Anchor, cable or equipment malfunctions or failures - Limited visibility or poor communication with crew | 3 Likely | 4 Critical | 12 High Risk | - Only qualified and trained personnel to work with cable winch systems and snowcats - Inspect cable, drum, and anchor points before each use - Keep exclusion zones around cable paths and prohibit workers crossing cables while under tension - Perform controlled tension test and adjust operation to snow and terrain conditions - Use tag lines or tools to guide cable ends - Avoid hands near pinch points and entrapment hazards - De-tension cable fully before release or disconnection | 2 Possible | 4 Critical | 8 Med. Risk | Confirm anchor certification and engineering requirements, never winch off trees or uncertified anchor points or overload the rated capacity Suspend or modify grooming operations when visibility and temperature fall below safe limits as part of weather monitoring and emergency plan |
| Musculoskeletal injuries (MSI) and ergonomics | - Repetitive motions when steering, operating controls, or using hand tools for maintenance - Awkward postures when climbing in/out of snowcats, reaching into engine or working under snowcat - Overexertion when lifting or moving heavy attachments, tracks, or maintenance parts - Pushing or pulling when handling hydraulic hoses, chains, or adjusting grooming attachments - Carrying tools, fuel, or parts across uneven terrain or into snowcats - Ergonomic strain from prolonged sitting, whole-body vibration, and awkward seating/controls | 3 Likely | 3 Serious | 9 High Risk | - Training on MSI prevention, ergonomics, safe lifting, shovelling, positioning, and tool-use, as applicable - Adjust seat height, backrest, armrests, and controls to maintain neutral posture and reduce vibration exposure during operation - Reminders during pre-shift briefings on MSI prevention - Warm-up, stretch and take breaks including micro-breaks - Rotate tasks throughout the shift, as applicable - Use proper lifting techniques or mechanical aids - Different sized shovels and ergonomic/extendable tools - Break heavy loads (e.g. sand/salt bags) into smaller loads - Use team lifts for awkward or heavy items - Use equipment with wheels, sliders, or handles, if possible - Push carts instead of pulling to reduce risk of MSIs - Inspect, maintain, and replace worn or broken equipment - Store heavier items at waist level to avoid unnecessary lifting - Encourage early reporting of discomfort or MSI symptoms | 2 Possible | 3 Serious | 6 Med. Risk | See resources for employers from WorkSafeBC and go2HR on MSI prevention See Road Safety BC for ergonomic resources associated with operating vehicles and equipment Encourage maintenance of physical fitness during off-season and throughout season |
| Severe weather and cold stress | - Cold stress injuries such as frostbite and hypothermia - Impaired decision-making and slower reaction times - Slips, trips, and falls from snow and ice accumulation on surfaces - Reduced visibility from blowing snow, fog, or whiteout conditions - Equipment malfunctions or failures caused by ice buildup or frozen components - Sun exposure (UV risks) can be increased with reflection from snow and at higher altitudes | 3 Likely | 3 Serious | 9 High Risk | - Training on cold stress prevention and reminders during briefings, including early warning signs of cold stress - Ensure staff dress in layers and insulated, waterproof and wind-resistant clothing, gloves, headgear and boots - Equip workers with high-vis and reflective outerwear - Schedule regular breaks and rotations in heated shelters - Provide warm, non-caffeinated drinks in break areas - Perform buddy-checks to monitor alertness and cold stress - Encourage proper rest, nutrition, and hydration before shifts - Keep work areas, platforms, and paths clear of snow/ice and apply sand, salt, grit, or traction aids to walking surfaces - Conduct frequent inspections and de-icing of equipment - Report and replace worn or broken equipment - Wear UV-rated sunscreen and eye protection, as applicable | 2 Possible | 3 Serious | 6 Med. Risk | See resources for employers from WorkSafeBC and go2HR on cold stress prevention Suspend or modify grooming operations when visibility and temperature fall below safe limits as part of weather monitoring and emergency plans |
| Overnight coverage and fatigue | - Negative impact to mental well-being and health - Slower reaction times or reduced ability increasing incident risks - Communication errors - Increased absenteeism | 3 Likely | 2 Moderate | 6 Med. Risk | - Ensure overnight tasks with limited visibility have adequate environmental and/or equipment lighting - Schedule regular breaks during overnight shifts - Limit shift length and avoid excessive overtime - Avoid scheduling critical tasks during low-alert periods - Train staff to recognize and report signs of fatigue - Encourage healthy sleep habits and work-life balance - Share mental health resources and support options - Supervisors to check-in to assess fit for duty and alertness | 2 Possible | 2 Moderate | 4 Med. Risk | See resources for employers from WorkSafeBC and go2HR on psychological safety, mental health and fatigue |