|  | Industry/Sector: | Ski Hill and Areas | Department/Job Position: | Snowmaking |
| --- | --- | --- | --- | --- |
|  | Common Tasks and Duties: | - Operating and monitoring snowmaking machines and compressors, including compressor plants - Connecting and disconnecting hoses, couplings, and hydrants - Connecting and disconnecting to generators and electrical systems and equipment - Moving and adjusting snow guns and nozzles - Working in proximity to snowcats and other mobile equipment - Performing maintenance and troubleshooting in variable terrain - Recording snowmaking output and system performance - Working alone at night in low-visibility conditions | Reasonably Foreseeable or Common Hazards: | - Slips, trips and falls, including tree wells, snow instability, embankments and other hill hazards - Moving/rotating parts and pinch point hazards - Struck by/against (e.g. snowcat, UTV, snowmobile) - High-pressurized water and air - Electrical hazards, including in wet environments - Noise and vibration hazards - Musculoskeletal injuries (MSI) and ergonomics - Harassment and violence, working alone - Severe weather and cold stress - Limited visibility during night work - Fatigue and psychological hazards |

Employers are to review this HIRA document and customize to their location, job tasks and site-specific hazards. To edit a row, simply use Microsoft word to edit (e.g. edit text, change risk ratings, delete a row, or add a new row). When determining risk follow the Guidance Document - Assessing Risk for HIRA Tools. When identifying controls, be sure to use the Hierarchy of Controls to control risk through elimination, substitution, engineering controls, administrative controls and Personal Protective Equipment (PPE). Once complete, this should be reviewed by the JHSC or a Worker H&S Rep, and reviewed annually thereafter.

|  | Full Name | Job Position | Signature | Date |
| --- | --- | --- | --- | --- |
| Completed By: |  |  |  |  |
| Reviewed By: |  |  |  |  |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Travelling on hill (e.g. snowmobile, snowcat, UTV or service vehicle) | - Slips, trips, and falls - Tree wells, snowpack instability, embankments and hill hazards - Collisions with objects, guests, workers, or equipment - Loss of control or rollover of snowmobile, snowcat, UTV or service vehicle on steep, icy, or uneven terrain - Struck by moving vehicles or equipment - Breakdown in remote area - Working alone and in isolation - Wildlife on trails or remote routes - MSI (strains/sprains) risks - Severe weather and cold stress | 3 Likely | 3 Serious | 9 High Risk | - Operator training and competency with skills assessment for snowmobile, UTV or service vehicle use - Follow PPE requirements for safety headgear, headlamp, eye protection, hearing protection and reflective materials - Use seatbelts or other safety restraints when installed - Operate within equipment manufacturer’s slope limitations - Proper footwear and traction aids (e.g. cleats, Yaktrax, mats) - Ensure designated routes, adherence to speed limits and prohibit cellphone use while operating vehicles/equipment - Conduct pre-use inspections of vehicles and equipment - Plan travel according to weather and avalanche conditions - Working Alone Policy with established check-in procedures - Load carrying and tow restrictions, equipment racks with procedures for securing equipment and use of siren/lights - Maintain situational awareness, avoid distractions - Training in wildlife awareness and safety, with deterrents - Devices for immediately summoning assistance (e.g. radios, apps, Sat. comms, inReach, GPS, panic button, whistle etc.) - Carry emergency kit and gear (e.g. first aid kit, lighting, blanket, survival pack, tow rope, extra fuel etc.) - Report/address unsafe operation (speeding, reckless driving, horseplay) or travel conditions (icy trails, poor signage) | 2 Possible | 3 Serious | 6 Med. Risk | Avoid work with increased risk when working alone. When possible, have staff work in pairs or groups, particularly in locations and with tasks that are increased risk Briefings to provide overview on daily hazards, expected weather, fit for duty, planned and alternate routes, safe work practices, PPE, seatbelt use, and rescue and emergency procedures Suspend or modify snowmaking operations when visibility and temperature fall below safe limits or other unsafe conditions as part of weather monitoring and emergency plans |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Clearing snow/ice from base, stations, towers, and equipment | - Slips, trips and falls - Falling ice/snow or other objects - Falls when using ladders or elevating work platforms - Severe weather and cold stress - Significant MSI risks due to heavy and rigorous movement associated with snow clearing and snowmaking activities | 3 Likely | 3 Serious | 9 High Risk | - Regular de-icing and snow removal to minimize risk of excessive snow and ice build-up or equipment failure - Training on proper shovelling and snow removal techniques - Different sized shovels and ergonomic/extendable tools - De-icing roof of huts and structures, regular snow clearing - Set up work areas to prevent tools/materials and ice/snow from falling and use work delineation (e.g. barriers, cones) - Proper footwear and traction aids (e.g. cleats, Yaktrax, mats) - Use wet floor signage to mark wet floors and slip hazards - Supervision to monitor correct techniques and controls | 2 Possible | 3 Serious | 6 Med. Risk | Use snowblowers only when trained and when safe to operate without creating risks to other workers or guests in the area |
| Assisting snowcat operations and working around mobile equipment | - Slips, trips and falls - Struck by or crushing hazards from moving snowcats or other mobile equipment - Struck by attachments (blades, rakes) or entanglement from winch cables - Limited operator visibility leading to “blind spots” - Noise and loud environments - Sprayed snow and accumulated snow piles - Severe weather and cold stress | 3 Likely | 3 Serious | 9 High Risk | - Establish clear work areas and exclusion zones - Only qualified and trained workers permitted in work zone - Provide training on potential blind spots and hand signals - Proper equipment and materials, including PPE (e.g. safety headgear, gloves, eye protection and high-vis clothing) - Ensure functioning of radios, Sat. comms/inReach devices - Pre-use inspections to ensure safety devices (e.g. lights, alarms, guards etc.) are operational - Schedule work during off-hours or controlled closures - Prohibit entry into areas when snowcats are operating and secure with signage and barriers to prevent entry - Approach mobile equipment after it has stopped, and only after eye contact with the operator and signal confirmation | 2 Possible | 3 Serious | 6 Med. Risk | Suspend or modify snowcat and other mobile equipment use when visibility and temperature fall below safe limits or other unsafe conditions as part of weather monitoring and emergency plans Ensure adequate lighting and visibility - if not possible, take additional precautions (e.g. spotters, closures etc.) |
| Working around moving parts and equipment (e.g. fans, belts, snow guns, lifting equipment) | - Struck-by hazards from rotating nozzles, fans, or snow guns - Entanglement or caught-in injuries from cables, fans, belts, pulleys, or drive shafts - Pinch points when adjusting valves, hoses, or couplings - Crush hazards when raising/lowering tower guns, or lifting equipment - Inadvertent/unexpected start-up of equipment - Noise and vibration hazards | 3 Likely | 3 Serious | 9 High Risk | - Train workers on recognition of pinch points, entanglement hazards, and safe approach distances - Use guarding and barriers around exposed moving parts - Ensure snow guns are properly positioned with clearance - De-energize equipment before performing maintenance - Lockout/tagout (LOTO) procedures with individual locks/tags - Follow manufacturer’s instruction for safe use, pre-use inspection and maintenance - Ensure communication and coordination before starting any work or when re-starting equipment after service - Avoid loose clothing, secure hair, jewellery, and ID badges - Maintain situational awareness, avoid distractions - Wear PPE (e.g. gloves, safety glasses, hearing protection) - Establish and train workers on emergency stop procedures | 2 Possible | 3 Serious | 6 Med. Risk | Suspend or modify snowmaking operations when visibility and temperature fall below safe limits or other unsafe conditions as part of weather monitoring and emergency plans |
| Using hand and power tools (e.g. drills, wrenches, snowblowers, snow guns, winches, portable heaters) | - Cuts, punctures, abrasions or eye injuries from tools - Rotating and moving parts - Burns from tiger torches or other heating and warming equipment - Faulty or defective equipment - Noise and vibration hazards - Electric shock from improperly maintained or insulated tools - Falls from step stools or ladders - MSI (strains/sprains) risks - Severe weather and cold stress | 3 Likely | 3 Serious | 9 High Risk | - Only qualified and trained personnel to work with equipment - Do not defeat or bypass safety guards or devices - Follow manufacturer’s instruction for safe use, pre-use inspection and maintenance, and safe work procedures - Ensure all power tools are double-insulated or properly grounded and rated for outdoor use - Wear PPE (e.g. gloves, safety glasses, hearing protection) - Avoid loose clothing, secure hair, jewellery, and ID badges - Maintain situational awareness, avoid distractions - Set up exclusion areas with work delineation (e.g. barriers, cones, signage etc.) to keep others out of the work areas - Store tools securely to prevent slips, trips, or drops - Set up work areas to prevent tools/materials from falling and use adequate work delineation (e.g. barriers, cones etc.) | 2 Possible | 3 Serious | 6 Med. Risk | Safe ladder use includes pre-use inspection of ladders and equipment, ensuring CSA or ANSI-certified, maintaining 3-points of contact, training, ensuring proper set-up and using work delineation and spotters, see WorkSafeBC Ladder Safety |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Performing hot work (e.g. welding, cutting, grinding, torch use) | - Burns from sparks, molten metal, or hot surfaces - Fire or explosion from sparks contacting combustible materials - Eye injuries from arc flash, flying discs, sparks, or debris - Respiratory hazards from welding fumes, grinding dust, or smoke - Noise and vibration hazards | 3 Likely | 3 Serious | 9 High Risk | - Only trained and qualified personnel to perform hot work - Hot work permit system in accordance with WorkSafeBC - Remove or shield combustible and flammable materials - Use required PPE (e.g. face shield, safety glasses, gloves, FR clothing, hearing protection, respirator etc.) - Ensure fire extinguishers are available with workers trained - Assign a fire watch during and after hot work - Use local exhaust ventilation or portable fume extractors - Restrict access to hot work areas using work delineation | 2 Possible | 3 Serious | 6 Med. Risk | If contracting work with increased risk, ensure contractor safety is managed through controls such as pre-qualification, confirmation of valid insurance (including WSBC clearance letter), hazard assessments, verification of qualifications and training, and safe work procedures and equipment |
| Working with hydraulic and pressurized snowmaking systems (e.g. water lines, air lines, hoses, couplings, hydrants, pumps, compressors, gas cylinders) | - Slips, trips and falls - High-pressure fluid, gas, or air release causing injection injuries or burns - Hose or fitting rupture, failure or malfunction releasing energy - Eye injuries from flying debris or materials - Crushing or pinching hazards - Noise and vibration hazards | 3 Likely | 3 Serious | 9 High Risk | - Only qualified and trained personnel to work with hydraulic and pressurized snowmaking systems and equipment - Follow manufacturer’s instruction for safe use, pre-use inspection and maintenance, and safe work procedures - De-energize equipment before performing maintenance, and relieve system pressure before disconnecting or servicing - Use guarding, blocks, or mechanical restraints to prevent unintended movement of parts - Lockout/tagout (LOTO) procedures with individual locks/tags - Use manufacturer-specified tools, fittings, and rated hoses to prevent leaks and failures, do not substitute with other parts - Inspect hoses, fittings, and cylinders regularly for wear, cracks, or damage, and replace immediately - Wear PPE (e.g. gloves, safety glasses, hearing protection), particularly when connecting hoses/operating snow guns | 2 Possible | 3 Serious | 6 Med. Risk | Maintain clear walkways, place hoses to avoid tripping, and apply salt, sand, grit or de-icing where needed Post clear rules around working with compressed air and ensure gas regulators are in place Report and repair leaks immediately, do not operate equipment with known hydraulic defects |
| Using chemicals and hazardous products (e.g. glycol/oil, lubricants, fuels) | - Exposure to chemical or biological hazards (e.g. chemical burns, splashes to eyes, respiratory or skin irritation) - Unlabelled or illegible containers - Spills and environment concerns | 2 Possible | 3 Serious | 6 Med. Risk | - WHMIS training for staff using chemical products - Supplier and workplace labels in place and legible - Safe handling and storage, see Safety Data Sheets (SDS) - Use PPE (e.g. gloves and eye protection) - Employer to assess products for hazards before procuring - Ensure adequate and good ventilation in working areas | 1 Unlikely | 2 Moderate | 2 Low Risk | Ensure first aid is on site in accordance with WorkSafeBC requirements, and first aid supplies including emergency eyewash Ensure spill response procedures in accordance with SDS |
| Operating and working with electrical systems for snowmaking (e.g. fans, snow guns, compressors, control panels, pumps, power cables) | - Electric shock from exposed or faulty wiring or components - Electrical fires or burns from arc flash or hot electrical parts - Eye injuries from arc flash, sparks, or flying debris - Faulty tools, extension cords, or portable equipment - Equipment not rated for outdoor winter weather conditions | 3 Likely | 3 Serious | 9 High Risk | - Only qualified and authorized personnel to perform electrical work, and provide workers with training on snowmaking equipment specific electrical hazards and procedures - De-energize equipment before performing maintenance - Lockout/tagout (LOTO) procedures with individual locks/tags - Use insulated tools, test equipment, and PPE rated for electrical tasks, and environmental conditions - Ensure electrical components, cords, and equipment are in good condition and rated for outdoor use in cold/wet weather - Keep generators and equipment dry and unobstructed - Conduct inspections of cords, plugs, connectors, panels, heaters, and control systems | 2 Possible | 3 Serious | 6 Med. Risk | Ensure proper grounding and bonding of electrical equipment including generators, temporary power and snow gun equipment Use, maintain and test Ground Fault Circuit Interrupters (GFCIs) where applicable |
| Overnight coverage and fatigue | - Negative impact to mental well-being and health - Slower reaction times or reduced ability increasing incident risks - Communication errors - Increased absenteeism | 3 Likely | 2 Moderate | 6 Med. Risk | - Ensure overnight tasks with limited visibility have adequate environmental and/or equipment lighting - Schedule regular breaks during overnight shifts - Limit shift length and avoid excessive overtime - Avoid scheduling critical tasks during low-alert periods - Train staff to recognize and report signs of fatigue - Encourage healthy sleep habits and work-life balance - Share mental health resources and support options - Supervisors to check-in to assess fit for duty and alertness | 2 Possible | 2 Moderate | 4 Med. Risk | See resources for employers from WorkSafeBC and go2HR on psychological safety, mental health and fatigue |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Musculoskeletal injuries (MSI) and ergonomics | - Repetitive motions from connecting hoses, operating valves, shovelling, or clearing ice/snow - Awkward postures when climbing towers, reaching nozzles, or crouching at hydrants/pits - Overexertion from lifting or moving heavy snow guns, hoses, or generators - Pushing or pulling frozen hoses, fan guns, or moving snowblowers - Carrying tools, parts, or hoses over snow/ice terrain | 3 Likely | 3 Serious | 9 High Risk | - Training on MSI prevention, ergonomics, safe lifting, shovelling, positioning, and tool-use for snowmaking - Reminders during pre-shift briefings on MSI prevention - Warm-up, stretch and take breaks including micro-breaks - Rotate tasks (e.g. shovelling) throughout the shift - Use proper lifting techniques or mechanical aids - Different sized shovels and ergonomic/extendable tools - Break heavy loads (e.g. sand/salt bags) into smaller loads - Use team lifts for awkward or heavy items - Use equipment with wheels, sliders, or handles, if possible - Push carts instead of pulling to reduce risk of MSIs - Inspect, maintain, and replace worn or broken equipment - Store heavier items at waist level to avoid unnecessary lifting - Encourage early reporting of discomfort or MSI symptoms | 2 Possible | 3 Serious | 6 Med. Risk | See resources for employers from WorkSafeBC and go2HR on MSI prevention Encourage maintenance of physical fitness during off-season and throughout season |
| Severe weather and cold stress | - Cold stress injuries such as frostbite and hypothermia - Impaired decision-making and slower reaction times - Slips, trips, and falls from snow and ice accumulation on surfaces - Reduced visibility from blowing snow, fog, or whiteout conditions - Equipment malfunctions or failures caused by ice buildup or frozen components - Sun exposure (UV risks) can be increased with reflection from snow and at higher altitudes | 3 Likely | 3 Serious | 9 High Risk | - Training on cold stress prevention and reminders during briefings, including early warning signs of cold stress - Ensure staff dress in layers and insulated, waterproof and wind-resistant clothing, gloves, headgear and boots - Equip workers with high-vis and reflective outerwear - Schedule regular breaks and rotations in heated shelters - Provide warm, non-caffeinated drinks in break areas and encourage staying hydrated - Perform buddy-checks to monitor alertness and cold stress - Encourage proper rest, nutrition, and hydration before shifts - Keep work areas, platforms, and paths clear of snow/ice and apply sand, salt, grit, or traction aids to walking surfaces - Conduct frequent inspections and de-icing of equipment - Report and replace worn or broken equipment - Wear UV-rated sunscreen and eye protection, as applicable | 2 Possible | 3 Serious | 6 Med. Risk | See resources for employers from WorkSafeBC and go2HR on cold stress prevention Suspend or modify snowmaking operations when visibility and temperature fall below safe limits as part of weather monitoring and emergency plans |