|  | Industry/Sector: | Ski Hill and Areas | Department/Job Position: | Base Operations |
| --- | --- | --- | --- | --- |
|  | Common Tasks and Duties: | - Directing and assisting guests in base areas, including ticket validation and customer service - Setting up, maintaining, and removing signage, fencing, barricades, and crowd management equipment - Parking management and traffic direction, - Snow removal, sanding, and salting of walkways, stairs, and parking lots - Road maintenance including plowing, sanding, and equipment operation (e.g. bobcat, skid steer, snowplow) - Operating shuttle buses and company vehicles - Cleaning and stocking washrooms and other base facilities - Handling and disposing of waste and recycling - Responding to minor incidents and summoning patrol/security | Reasonably Foreseeable or Common Hazards: | - Slips, trips and falls - Collisions with guests, coworkers, or equipment - Struck by/against (snowcat, UTV, snowmobile) - Moving/rotating parts pinch point hazards (machinery/equipment) - Electrical, noise and vibration hazards - Working at heights - Falling objects (e.g. ice, snow, guest items) - Chemical exposure from cleaning agents, fuel etc. - Musculoskeletal injuries (MSI) and ergonomics - Harassment and violence, cash handling - Severe weather and cold stress - Fatigue and psychological hazards, working alone |

Employers are to review this HIRA document and customize to their location, job tasks and site-specific hazards. To edit a row, simply use Microsoft word to edit (e.g. edit text, change risk ratings, delete a row, or add a new row). When determining risk follow the Guidance Document - Assessing Risk for HIRA Tools. When identifying controls, be sure to use the Hierarchy of Controls to control risk through elimination, substitution, engineering controls, administrative controls and Personal Protective Equipment (PPE). Once complete, this should be reviewed by the JHSC or a Worker H&S Rep, and reviewed annually thereafter.

|  | Full Name | Job Position | Signature | Date |
| --- | --- | --- | --- | --- |
| Completed By: |  |  |  |  |
| Reviewed By: |  |  |  |  |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Operating service vehicles, busses/shuttles, snowplows, or equipment (e.g. bobcats or UTVs) | - Slips, trips, and falls - Collisions with objects, guests, workers, or equipment - Struck by moving vehicles or equipment - Loss of control, rollover or breakdown of vehicles and equipment on steep or icy terrain - Traffic management hazards - Wildlife collisions - Working alone and in isolation - Noise and vibration hazards - MSI (strains/sprains) risks - Severe weather and cold stress - Working overnight and fatigue - Harassment and violence when operating guest busses or shuttles | 3 Likely | 3 Serious | 9 High Risk | - Operator training and competency for operating vehicles and equipment, confirmation of valid licence for vehicle class - Proper footwear and traction aids (e.g. cleats, Yaktrax, mats) - Ensure designated routes, winter tires, adherence to speed limits, and seatbelt use. Prohibit cellphone use - Conduct pre-use inspections of vehicles and equipment - Plan travel according to weather and traffic conditions - Working Alone Policy with established check-in procedures - Load carrying and tow restrictions, equipment racks with procedures for securing equipment and use of siren/lights - Maintain situational awareness, avoid distractions - Use spotters in areas with pedestrian traffic and congestion - Devices for immediately summoning assistance (e.g. radios, apps, Sat. comms, inReach, GPS, panic button, whistle etc.) - Carry emergency kit and gear (e.g. first aid kit, lighting, cones, blanket, tow rope, extra fuel, tire chains etc.) - Report/address unsafe operation (speeding, reckless driving, horseplay) or travel conditions (icy roads, poor visibility) - See HIRA for control measures for MSI and severe weather | 2 Possible | 3 Serious | 6 Med. Risk | Avoid work with increased risk when working alone. When possible, have staff work in pairs or groups, particularly in locations and with tasks that are increased risk Briefings to provide overview on daily hazards, expected weather, fit for duty, planned and alternate routes, safe work practices, PPE, and rescue/emergency procedures Follow PPE requirements for high-vis clothing, safety headgear, headlamp, eye protection, hearing protection and reflective materials, as applicable |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Clearing snow/ice from base areas, guest and customer services, rentals etc. | - Slips, trips and falls - Falling ice/snow or other objects - Falls when using ladders or elevating work platforms - Severe weather and cold stress - Significant MSI risks due to heavy and rigorous movement associated with snow clearing activities | 3 Likely | 3 Serious | 9 High Risk | - Regular de-icing and snow removal to minimize risk of excessive snow and ice build-up or equipment failure - Training on proper shovelling and snow removal techniques - Different sized shovels and ergonomic/extendable tools - De-icing roof of huts and structures, regular snow clearing - Set up work areas to prevent tools/materials and ice/snow from falling and use work delineation (e.g. barriers, cones) - Proper footwear and traction aids (e.g. cleats, Yaktrax, mats) - Use wet floor signage to mark wet floors and slip hazards - Supervision to monitor correct techniques and controls | 2 Possible | 3 Serious | 6 Med. Risk | Use snowblowers only when trained and when safe to operate without creating risks to other workers or guests in the area See HIRA for information on working from heights and safe ladder use |
| Pre-opening checks and setting up line control | - Slips, trips and falls - Falling ice/snow or other objects - Falls when using step stools - Working alone and in isolation - MSI (strains/sprains) risks - Severe weather and cold stress - Harassment and violence | 2 Possible | 3 Serious | 6 Med. Risk | - Proper set-up, spacing and shoots for line control and barriers, based on anticipated attendance and use - Physical barriers (e.g. ropes, fencing, cones, delineators), ensuring ability for emergency access if needed - Applying sand, grit and salt and traction aids, as applicable - Clear signage to identify lineup etiquette and structure - Visible staff with polite and direct enforcement of rules and line management to guests with extra staff at peak times - Maintain situational awareness and crowd monitoring | 2 Possible | 2 Moderate | 4 Med. Risk | Briefings to provide overview on daily hazards, expected weather, fit for duty, planned and alternate routes, safe work practices, PPE, and rescue/emergency procedures |
| Parking management and traffic direction including enforcement | - Slips, trips and falls - Falling ice/snow or other objects - Struck by moving vehicles or equipment - Traffic management hazards - Wildlife in the area - Working alone and in isolation - Noise and vibration hazards - MSI (strains/sprains) risks - Severe weather and cold stress - Harassment and violence, including cash handling | 3 Likely | 3 Serious | 9 High Risk | - Training on hazards and traffic management requirements - Ensure regular snow removal and de-icing along with clear traffic management signage and instruction - Follow PPE requirements for high-vis clothing, safety footwear, headlamp, eye protection, hearing protection and reflective materials, as applicable - Use spotters in areas with pedestrian traffic and congestion - Proper footwear and traction aids (e.g. cleats, Yaktrax, mats) - Working Alone Policy with established check-in procedures - Devices for immediately summoning assistance (e.g. radios, apps, Sat. comms, inReach, GPS, panic button, whistle etc.) - See HIRA for control measures for harassment and violence | 2 Possible | 3 Serious | 6 Med. Risk | Consult traffic management experts for layout and design of vehicle routes and parking Traffic management and control to comply with WorkSafeBC requirements on Traffic Control |
| Handling and disposing of waste and recycling | - Exposure to biological hazards e.g. blood, bodily fluids - Sharps e.g. syringes and needles - Broken glass - Increased risk due to wildlife - Crush or pinch point hazards associated with compactor - MSI (strains/sprains) risks | 3 Likely | 3 Serious | 9 High Risk | - Wildlife and tamper proof waste containers - Training on hazards and safe waste disposal requirements - Follow PPE requirements for high-vis clothing, cut and puncture resistant gloves and safety footwear, as applicable - Inspect the area, remove objects that pose tripping hazards - Promptly clean up snow/ice or spills, use “Wet Floor” signs - Ensure functioning of radios, Sat. comms/inReach devices - Equipment for sharps/glass disposal e.g. tongs, puncture-resistant gloves, sharps biohazard container etc. - Training on use of compactor to ensure no manual clearing - See HIRA for control measures for MSI and severe weather | 2 Possible | 3 Serious | 6 Med. Risk | Compactor to have signage, exclusion zones, no exposed parts (e.g. guards and barriers in place) and trained individuals with de-energization and Lockout/Tagout (LOTO) procedures for maintenance or repair work |
| Cleaning and restocking washrooms and facilities | - Exposure to biological hazards e.g. blood, bodily fluids - Sharps e.g. syringes and needles - MSI (strains/sprains) risks - Harassment and violence - Exposure to chemical cleaning products and hazardous products | 2 Possible | 3 Serious | 6 Med. Risk | - Close washrooms and facility areas during cleaning - Inspect the area, remove objects that pose tripping hazards - Promptly clean up snow/ice or spills, use “Wet Floor” signs - Ensure functioning of radios, Sat. comms/inReach devices - Written procedure for handling blood/bodily fluids and sharps - Training on sharps, biohazards and blood/bodily fluids - Follow PPE requirements for gloves, eye/face protection etc. - Proper footwear and traction aids (e.g. cleats, Yaktrax, mats) - Maintain situational awareness, avoid distractions | 2 Possible | 2 Moderate | 4 Med. Risk | Should blood/bodily fluid and sharps be identified, workers are to report to the supervisor to have designated and trained personnel with equipment and PPE to clean and dispose of blood/bodily fluid and sharps |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Using chemicals and hazardous products (e.g. chemical cleaners) | - Exposure to chemical hazards (e.g. chemical burns, splashes to eyes, respiratory or skin irritation) - Unlabelled or illegible containers - Spills and environment concerns | 2 Possible | 3 Serious | 6 Med. Risk | - WHMIS training for staff using chemical products - Supplier and workplace labels in place and legible - Safe handling and storage, see Safety Data Sheets (SDS) - Follow PPE requirements for gloves, eye/face protection etc. - Employer to assess products for hazards before procuring - Ensure adequate and good ventilation in working areas - Regular inspections for WHMIS compliance - Ensure proper storage of cleaners in secured area or cart | 2 Possible | 2 Moderate | 4 Med. Risk | Ensure first aid is on site in accordance with WorkSafeBC requirements, and first aid supplies including emergency eyewash Ensure spill response procedures in accordance with SDS |
| Using hand and power tools (e.g. drills, wrenches, snowblowers, portable heaters) and ladders when working from heights | - Cuts, punctures, abrasions or eye injuries from tools - Rotating and moving parts - Burns from tiger torches or other heating and warming equipment - Faulty or defective equipment - Noise and vibration hazards - Electric shock from improperly maintained or insulated tools - Falls from step stools or ladders - MSI (strains/sprains) risks - Severe weather and cold stress | 3 Likely | 3 Serious | 9 High Risk | - Only qualified and trained personnel to use tools/equipment - Do not defeat or bypass safety guards or devices - Follow manufacturer’s instruction for safe use, pre-use inspection and maintenance, and safe work procedures - Ensure all power tools are double-insulated or grounded - Inspect cords, panels, and equipment to ensure in good condition, and rated for outdoor use in cold and wet weather - Keep cords and hoses organized and out of walkways - Wear PPE (e.g. gloves, safety glasses, hearing protection) - Avoid loose clothing, secure hair, jewellery, and ID badges - Maintain situational awareness, avoid distractions - Set up exclusion areas with work delineation (e.g. barriers, cones, signage etc.) to keep others out of the work areas - Store tools securely to prevent slips, trips, or drops - Set up work areas to prevent tools/materials from falling and use adequate work delineation (e.g. barriers, cones etc.) | 2 Possible | 3 Serious | 6 Med. Risk | Safe ladder use includes pre-use inspection of ladders and equipment, ensuring CSA or ANSI-certified, maintaining 3-points of contact, training, ensuring proper set-up and using work delineation and spotters, see WorkSafeBC Ladder Safety |
| Musculoskeletal injuries (MSI) and ergonomics | - Repetitive motions from cleaning, shovelling, or using equipment - Awkward postures when lifting bins, bending, or stocking items - Overexertion when lifting or moving heavy garbage bins, barriers, salt bags, or equipment - Pushing or pulling carts, barriers, garbage bins, or snow shovels - Carrying loads such as signs, tools, supplies, or bags of salt/sand over icy/uneven terrain - Ergonomic strain from prolonged sitting, whole-body vibration, and awkward seating/controls when operating vehicles/equipment | 3 Likely | 3 Serious | 9 High Risk | - Training on MSI prevention, ergonomics, safe lifting, shovelling, positioning, and tool-use for base operations - Adjust seat height, backrest, armrests, and controls to maintain neutral posture and reduce vibration exposure during operation for vehicles and equipment - Reminders during pre-shift briefings on MSI prevention - Warm-up, stretch and take breaks including micro-breaks - Rotate tasks (e.g. shovelling) throughout the shift - Use proper lifting techniques or mechanical aids - Different sized shovels and ergonomic/extendable tools - Break heavy loads (e.g. sand/salt bags) into smaller loads - Use team lifts for awkward or heavy items - Use equipment with wheels, sliders, or handles, if possible - Push carts instead of pulling to reduce risk of MSIs - Inspect, maintain, and replace worn or broken equipment - Store heavier items at waist level to avoid unnecessary lifting - Encourage early reporting of discomfort or MSI symptoms | 2 Possible | 3 Serious | 6 Med. Risk | See resources for employers from WorkSafeBC and go2HR on MSI prevention See Road Safety BC for ergonomic resources associated with operating vehicles and equipment Encourage maintenance of physical fitness during off-season and throughout season |
| Harassment and violence due to working with the public – particularly when operating guest busses and shuttles | - Verbal abuse during long lines, delays, or closures - Threatening or aggressive behaviour when enforcing rules - Physical altercations (pushing, grabbing, striking) - Sexual harassment (comments, gestures, unwanted contact) - Stress from repeated or escalating incidents | 2 Possible | 3 Serious | 6 Med. Risk | - Harassment and violence prevention program in accordance with WorkSafeBC requirements - Training and awareness on harassment, violence, conflict resolution, de-escalation, and recognizing warning signs - Use dashcams for guest busses and shuttles - Clear rules and signage for rider’s code of conduct - Reporting procedures for response and incident reports - Increased security or patrol presence during peak periods - Support resources such as post-incident debriefing, mental health supports, or employee assistance programs | 2 Possible | 2 Moderate | 4 Med. Risk | See resources for employers from WorkSafeBC and go2HR on workplace harassment and violence prevention |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Handling cash, cash equivalents and payment systems | - Harassment and violence - Theft or robbery - Psychological safety and fatigue - Working alone and in isolation | 2 Possible | 3 Serious | 6 Med. Risk | - Provide workers with instruction and training - Security measures based on risk as part of violence risk assessment such as controlled access, panic buttons, CCTV, counters/barriers/safety glass, strategic layout (e.g. visible, unobstructed egress, lighting), alarm systems, cash handling protocols with secure drop safe, security guard presence and patrols, supervisor presence etc. | 1 Unlikely | 3 Serious | 3 Low Risk | See HIRA for more on harassment and violence if working with the public Consider cashless |
| Severe weather and cold stress | - Cold stress injuries such as frostbite and hypothermia - Impaired decision-making and slower reaction times - Slips, trips, and falls from snow and ice accumulation on surfaces - Reduced visibility from blowing snow, fog, or whiteout conditions - Equipment malfunctions or failures caused by ice buildup or frozen components - Sun exposure (UV risks) can be increased with reflection from snow and at higher altitudes | 3 Likely | 3 Serious | 9 High Risk | - Training on cold stress prevention and reminders during briefings, including early warning signs of cold stress - Ensure staff dress in layers and insulated, waterproof and wind-resistant clothing, gloves, headgear and boots - Equip workers with high-vis and reflective outerwear - Schedule regular breaks and rotations in heated shelters - Provide warm, non-caffeinated drinks in break areas and encourage staying hydrated - Perform buddy-checks to monitor alertness and cold stress - Encourage proper rest, nutrition, and hydration before shifts - Keep work areas, platforms, and paths clear of snow/ice and apply sand, salt, grit, or traction aids to walking surfaces - Conduct frequent inspections and de-icing of equipment - Report and replace worn or broken equipment - Wear UV-rated sunscreen and eye protection, as applicable | 2 Possible | 3 Serious | 6 Med. Risk | See resources for employers from WorkSafeBC and go2HR on cold stress prevention Suspend or modify base operations when visibility and temperature fall below safe limits as part of weather monitoring and emergency plans |
| Overnight coverage and fatigue | - Negative impact to mental well-being and health - Slower reaction times or reduced ability increasing incident risks - Communication errors - Increased absenteeism | 3 Likely | 2 Moderate | 6 Med. Risk | - Ensure overnight tasks with limited visibility have adequate environmental and/or equipment lighting - Schedule regular breaks during overnight shifts - Limit shift length and avoid excessive overtime - Avoid scheduling critical tasks during low-alert periods - Train staff to recognize and report signs of fatigue - Encourage healthy sleep habits and work-life balance - Share mental health resources and support options - Supervisors to check-in to assess fit for duty and alertness | 2 Possible | 2 Moderate | 4 Med. Risk | See resources for employers from WorkSafeBC and go2HR on psychological safety, mental health and fatigue |