|  | Industry/Sector: | Ski Hill and Areas | Department/Job Position: | Fleet Maintenance |
| --- | --- | --- | --- | --- |
|  | Common Tasks and Duties: | - Inspecting, repairing, and maintaining snowcats, snowmobiles, ATVs, trucks, and other vehicles - Logistics including scheduling and dispatch of fleet equipment - Performing preventative maintenance and refueling - Welding, grinding, and other hot work in the shop - Conducting tire or track changes and repairs - Using tools, hoists, jacks, and lifting devices to service fleet - Servicing engines, hydraulics, electrical, and fuel systems - Ordering, handling, and storing parts, oils, fuels, and chemicals - Coordinating with grooming, snowmaking, and patrols - Completing maintenance records and service logs - Responding to equipment failures in the field, and remote areas | Reasonably Foreseeable or Common Hazards: | - Slips, trips and falls - Struck by/against (snowcat, UTV, snowmobile) - Moving/rotating parts and pinch point hazards - Mechanical, hydraulic and electrical hazards - Noise and vibration hazards - Working at heights - Falling objects (e.g. ice, snow, guest items) - Chemical exposure from fuels, oils, solvents etc. - Musculoskeletal injuries (MSI) and ergonomics - Harassment and violence, working alone - Severe weather and cold stress - Fatigue and psychological hazards |

Employers are to review this HIRA document and customize to their location, job tasks and site-specific hazards. To edit a row, simply use Microsoft word to edit (e.g. edit text, change risk ratings, delete a row, or add a new row). When determining risk follow the Guidance Document - Assessing Risk for HIRA Tools. When identifying controls, be sure to use the Hierarchy of Controls to control risk through elimination, substitution, engineering controls, administrative controls and Personal Protective Equipment (PPE). Once complete, this should be reviewed by the JHSC or a Worker H&S Rep, and reviewed annually thereafter.

|  | Full Name | Job Position | Signature | Date |
| --- | --- | --- | --- | --- |
| Completed By: |  |  |  |  |
| Reviewed By: |  |  |  |  |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Operating equipment and vehicles (e.g. snowmobile, snowcat, UTV, busses, or service vehicles) | - Slips, trips, and falls - Tree wells, snowpack instability, embankments and hill hazards - Collisions with objects, guests, workers, or equipment - Struck by moving vehicles or equipment - Loss of control, rollover or breakdown of vehicles and equipment on steep or icy terrain - Traffic management hazards - Wildlife on trails or remote routes - Working alone and in isolation - Noise and vibration hazards - MSI (strains/sprains) risks - Severe weather and cold stress - Working overnight and fatigue | 3 Likely | 3 Serious | 9 High Risk | - Operator training and competency for operating vehicles and equipment, confirmation of valid licence for vehicle class - Ongoing defensive driving training and skills assessment - Proper footwear and traction aids (e.g. cleats, Yaktrax, mats) - Ensure designated routes, winter tires, adherence to speed limits, wear seatbelts, and prohibit cellphone use - Conduct pre-use inspections of vehicles and equipment - Plan travel according to weather and traffic conditions - Working Alone Policy with established check-in procedures - Load carrying and tow restrictions, equipment racks with procedures for securing equipment and use of siren/lights - Maintain situational awareness, avoid distractions - Use spotters in areas with pedestrian traffic and congestion - Devices for immediately summoning assistance (e.g. radios, apps, Sat. comms, inReach, GPS, panic button, whistle etc.) - Carry emergency kit and gear (e.g. first aid kit, lighting, cones, blanket, tow rope, extra fuel, tire chains etc.) - Report/address unsafe operation (speeding, reckless driving, horseplay) or travel conditions (icy roads, poor visibility) - See HIRA for control measures for MSI and severe weather | 2 Possible | 3 Serious | 6 Med. Risk | Avoid work with increased risk when working alone. When possible, have staff work in pairs or groups, particularly in locations and with tasks that are increased risk Briefings to provide overview on daily hazards, expected weather, fit for duty, planned and alternate routes, safe work practices, PPE, and rescue/emergency procedures Suspend or modify fleet maintenance operations when visibility and temperature fall below safe limits or other unsafe conditions as part of weather monitoring and emergency plans |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Responding to breakdowns in remote locations including performing work in variable terrain and/or performing tow activities | - Slips, trips, and falls - Tree wells, snowpack instability, embankments and hill hazards - Collisions with objects, guests, workers, or equipment - Entanglement, pinch point or caught-in hazards - Struck-by hazards from moving tracks, attachments, cables, and mechanical components - Broken cables or towing equipment, loss of control or rollover on steep or icy terrain - Wildlife on trails or remote routes - MSI (strains/sprains) risks - Severe weather and cold stress | 3 Likely | 4 Critical | 12 High Risk | - Inspect tow, cable, and anchor points before each use - Perform controlled tension test and adjust operations to specific equipment, snow and terrain conditions - Use tag lines or tools to guide cable ends, avoid hands near pinch points and entrapment hazards - Train workers on recognition of pinch points, entanglement hazards, and safe approach distances - Use guarding and barriers around exposed moving parts - De-energize equipment before performing maintenance - Lockout/tagout (LOTO) procedures with individual locks/tags - Follow manufacturer’s instruction for safe use, pre-use inspection and maintenance - Follow PPE requirements for safety headgear, headlamp, eye protection, hearing protection and reflective materials - Safety footwear and traction aids (e.g. cleats, Yaktrax, mats) - Avoid loose clothing, secure hair, jewellery, and ID badges - Maintain situational awareness, avoid distractions - Devices for immediately summoning assistance (e.g. radios, apps, Sat. comms, inReach, GPS, panic button, whistle etc.) - Training in wildlife awareness and safety, with deterrents | 2 Possible | 4 Critical | 8 Med. Risk | Equip fleet assets with GPS-based location tracking Confirm anchor certification and engineering requirements, never winch off trees or uncertified anchor points or overload the rated capacity Ensure fleet team is trained, regular drills conducted, and emergency tow materials and kit are inspected and maintained Carry emergency kit and gear (e.g. first aid kit, lighting, cones, blanket, tow rope, extra fuel, tire chains etc.) |
| Working in maintenance shop and around fleet vehicles and mobile equipment | - Slips, trips and falls - Struck by or crushing hazards from moving vehicles or other mobile equipment - Struck by attachments (blades, rakes, tillers) or entanglement from cables - Limited operator visibility leading to “blind spots” - Noise and loud environments - MSI (strains/sprains) risks | 3 Likely | 3 Serious | 9 High Risk | - Establish clear work areas and exclusion zones - Inspect the area, remove objects that pose tripping hazards - Promptly clean up snow/ice or spills, use “Wet Floor” signs - Ensure spill response procedures in accordance with SDS - Only qualified and trained workers permitted in work zone - Proper equipment and materials, including PPE (e.g. safety footwear, gloves, eye protection and high-vis clothing) - Ensure functioning of radios, Sat. comms/inReach devices - Pre-use inspections to ensure safety devices (e.g. mirrors, lights, alarms, guards etc.) are operational - Approach mobile equipment after it has stopped, and only after eye contact with the operator and signal confirmation | 2 Possible | 3 Serious | 6 Med. Risk | Post signage identifying loud environments and hearing protection required Review HIRA for specific tasks and hazards/risks when working in maintenance shop e.g. use of chemicals/fuels, performing work on electrical, hydraulic and mechanical components, hot work, and using tools and equipment such as hoists and jacks |
| Clearing snow/ice from areas and equipment | - Slips, trips and falls - Falling ice/snow or other objects - Falls when using ladders or elevating work platforms - Severe weather and cold stress - Significant MSI risks due to heavy and rigorous movement associated with snow clearing activities | 3 Likely | 3 Serious | 9 High Risk | - Regular de-icing and snow removal to minimize risk of excessive snow and ice build-up or equipment failure - Training on proper shovelling and snow removal techniques - Different sized shovels and ergonomic/extendable tools - De-icing roof of huts and structures, regular snow clearing - Set up work areas to prevent tools/materials and ice/snow from falling and use work delineation (e.g. barriers, cones) - Safety footwear and traction aids (e.g. cleats, Yaktrax, mats) - Use wet floor signage to mark wet floors and slip hazards - Supervision to monitor correct techniques and controls | 2 Possible | 3 Serious | 6 Med. Risk | Use snowblowers only when trained and when safe to operate without creating risks to other workers or guests in the area See HIRA for information on working from heights and safe ladder use |
| Performing hot work (e.g. welding, cutting, grinding, torch use) | - Burns from sparks, molten metal, or hot surfaces - Fire or explosion from sparks contacting combustible materials - Eye injuries from arc flash, flying discs, sparks, or debris - Respiratory hazards from welding fumes, grinding dust, or smoke - Noise and vibration hazards | 3 Likely | 3 Serious | 9 High Risk | - Only trained and qualified personnel to perform hot work - Hot work permit system in accordance with WorkSafeBC - Remove or shield combustible and flammable materials - Use required PPE (e.g. face shield, safety glasses, gloves, FR clothing, hearing protection, respirator etc.) - Ensure fire extinguishers are available with workers trained - Assign a fire watch during and after hot work - Use local exhaust ventilation or portable fume extractors - Restrict access to hot work areas using work delineation | 2 Possible | 3 Serious | 6 Med. Risk | If contracting work with increased risk, ensure contractor safety is managed through controls such as pre-qualification, confirmation of valid insurance (including WSBC clearance letter), hazard assessments, verification of qualifications and training, and safe work procedures and equipment |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Using chemicals and hazardous products (e.g. fuels, coolants, oils, lubricants, batteries, washers, solvents) | - Exposure to chemical hazards (e.g. chemical burns, splashes to eyes, respiratory or skin irritation) - Corrosion from battery acid and other corrosives - Flammables and fires, including increased risk from oily rags - Unlabelled or illegible containers - Spills and environment concerns | 2 Possible | 3 Serious | 6 Med. Risk | - WHMIS training for staff using chemical products - Supplier and workplace labels in place and legible - Safe handling and storage, see Safety Data Sheets (SDS) - Follow PPE requirements for gloves, eye/face protection etc. - Ensure no smoking and safe work procedures for refuelling - Fuel tanks to be properly stored with bonding/grounding and flame arrestors with workers trained in safe use - Use labelled, approved, self-closing metal containers for oily rag disposal and store away from ignition sources - Employer to assess products for hazards before procuring - Ensure spill response procedures in accordance with SDS | 2 Possible | 2 Moderate | 4 Med. Risk | Ensure first aid is on site in accordance with WorkSafeBC requirements, and first aid supplies including emergency eyewash Ensure adequate and good ventilation in working areas – if respirator required, ensure fit-testing and correct cartridges for the contaminant |
| Operating and working with electrical systems (e.g. tools and equipment, charging equipment for Electric Vehicles (EV), generators and batteries) | - Electric shock from exposed or faulty wiring or components - Electrical fires or burns from arc flash or hot electrical parts - Eye injuries from arc flash, sparks, or flying debris - Faulty tools, extension cords, or portable equipment - Equipment not rated for outdoor winter weather conditions | 3 Likely | 3 Serious | 9 High Risk | - Only qualified and authorized personnel to perform electrical work, and provide workers with training on fleet vehicles and equipment specific electrical hazards and procedures - De-energize equipment before performing maintenance - Lockout/tagout (LOTO) procedures with individual locks/tags - Use insulated tools, test equipment, and PPE rated for electrical tasks, and environmental conditions - Ensure electrical components, cords, and equipment are in good condition and rated for outdoor use in cold/wet weather - Use, maintain, and test Ground Fault Circuit Interrupters (GFCIs) where applicable - Keep generators and equipment dry and unobstructed - Conduct inspections of cords, plugs, connectors, panels, and other electrical equipment | 2 Possible | 3 Serious | 6 Med. Risk | Maintenance and repair of EVs requires proper training, use of insulated tools and PPE, and strict adherence to manufacturer instructions to prevent electrical incidents and fires Ensure proper grounding and bonding of electrical equipment including generators, temporary power and charging stations |
| Working with hydraulic and pressurized (e.g. equipment, hydraulic systems, compressed air, pressurized liquids, fuels and gases) | - Slips, trips and falls - High-pressure fluid, gas, or air release causing injection injuries or burns - Hose or fitting rupture, failure or malfunction releasing energy - Eye injuries from flying debris or materials - Crush or pinch point hazards - MSI (strains/sprains) risks - Noise and vibration hazards | 3 Likely | 3 Serious | 9 High Risk | - Only qualified and trained personnel to work with hydraulic and pressurized systems and equipment - Follow manufacturer’s instruction for safe use, pre-use inspection and maintenance, and safe work procedures - De-energize equipment before performing maintenance, and relieve system pressure before disconnecting or servicing - Use guarding, blocks, or mechanical restraints to prevent unintended movement of parts - Lockout/tagout (LOTO) procedures with individual locks/tags - Use manufacturer-specified tools, fittings, and rated hoses to prevent leaks and failures, do not substitute with other parts - Inspect hoses, fittings, and cylinders regularly for wear, cracks, or damage, and replace immediately - Wear PPE (e.g. gloves, safety glasses, hearing protection) | 2 Possible | 3 Serious | 6 Med. Risk | Post clear rules around working with compressed air and ensure gas regulators are in place Report and repair leaks immediately, do not operate equipment with known hydraulic defects |
| Using jacks, hoists, lifting devices and working around moving parts | - Inadvertent/unexpected start-up of equipment - Struck-by, entanglement, caught-in and pinch point hazards - Crush hazards when raising/lowering jacks, hoists or lifting equipment - Noise and vibration hazards | 3 Likely | 3 Serious | 9 High Risk | - Train workers on recognition of pinch points, crush, caught in and entanglement hazards, and safe approach distances - Avoid loose clothing, secure hair, jewellery, and ID badges - Use guarding and barriers around exposed moving parts - Ensure jacks and hoists and lifting equipment are only operated by trained individuals with the required function and capacity in accordance with manufacturer’s instructions - De-energize equipment before performing maintenance - Lockout/tagout (LOTO) procedures with individual locks/tags - Follow manufacturer’s instruction for safe use, pre-use inspection and maintenance - Ensure communication and coordination before starting any work or when re-starting equipment after service | 2 Possible | 3 Serious | 6 Med. Risk | Suspend or modify fleet maintenance operations when visibility and temperature fall below safe limits or other unsafe conditions as part of weather monitoring and emergency plans |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Using hand and power tools (e.g. drills, bearing press, wrenches, winches, torches) and handling scrap metal | - Cuts, punctures, or eye injuries from tools or scrap metal - Rotating and moving parts - Burns from tiger torches - Faulty or defective equipment - Noise and vibration hazards - Electric shock from improperly maintained or insulated tools - MSI (strains/sprains) risks | 3 Likely | 3 Serious | 9 High Risk | - Only qualified and trained personnel to work with tools - Do not defeat or bypass safety guards or devices - Follow manufacturer’s instruction for safe use, pre-use inspection and maintenance, and safe work procedures - Ensure all power tools are double-insulated or properly grounded and rated for outdoor use - Wear PPE (e.g. gloves, safety glasses, hearing protection) - Avoid loose clothing, secure hair, jewellery, and ID badges - Maintain situational awareness, avoid distractions | 2 Possible | 3 Serious | 6 Med. Risk | When handling scrap metal, workers must use gloves and mechanical aids to prevent cuts, punctures, and strains, and ensure material is placed in designated containers for safe storage and disposal |
| Working from heights (e.g. accessing machines and equipment with ladders or lifts) | - Slips, trips and falls - Falls from height when climbing into equipment or using ladders - Falling tools or materials posing risk to others below - Overreaching or loss of balance while working at heights - Contact with overhead hazards (e.g. overhead fan or equipment) | 3 Likely | 4 Critical | 12 High Risk | - Utilize the fall protection hierarchy to eliminate or guard against fall hazards as the preferred methods - Only qualified and trained personnel to work from heights - Follow WorkSafeBC requirements for fall protection - Maintain three points of contact while climbing/demounting - Face the machine and use designated step and handrails - Do not carry materials in your hand, use a backpack - Set up work areas to prevent tools/materials from falling and use adequate work delineation (e.g. barriers, cones etc.) | 2 Possible | 3 Serious | 6 Med. Risk | Safe ladder use includes pre-use inspection of ladders and equipment, ensuring CSA or ANSI-certified, maintaining 3-points of contact, training, ensuring proper set-up and using work delineation and spotters, see WorkSafeBC Ladder Safety |
| Musculoskeletal injuries (MSI) and ergonomics | - Repetitive motions from using hand tools, wrenches, snowblowers or power tools - Awkward postures when reaching into engines, working under vehicles, or crouching - Overexertion when lifting or moving heavy parts, tires, batteries, or components - Pushing or pulling equipment, tool carts, or equipment into shop - Carrying tools, parts or waste across uneven or icy terrain | 3 Likely | 3 Serious | 9 High Risk | - Training on MSI prevention and ergonomics - Reminders during pre-shift briefings on MSI prevention - Warm-up, stretch and take breaks including micro-breaks - Rotate tasks (e.g. awkward postures) throughout the shift - Use proper lifting techniques or mechanical aids - Different sized shovels and ergonomic/extendable tools - Break heavy loads (e.g. sand/salt bags) into smaller loads - Use team lifts for awkward or heavy items - Use equipment with wheels, sliders, or handles, if possible - Push carts instead of pulling to reduce risk of MSIs - Inspect, maintain, and replace worn or broken equipment - Store heavier items at waist level to avoid unnecessary lifting - Encourage early reporting of discomfort or MSI symptoms | 2 Possible | 3 Serious | 6 Med. Risk | See resources for employers from WorkSafeBC and go2HR on MSI prevention See Road Safety BC for ergonomic resources associated with operating vehicles and equipment Encourage maintenance of physical fitness during off-season and throughout season |
| Severe weather and cold stress | - Cold stress injuries such as frostbite and hypothermia - Impaired decision-making and slower reaction times - Slips, trips, and falls from snow and ice accumulation on surfaces - Reduced visibility from blowing snow, fog, or whiteout conditions - Equipment malfunctions or failure due to frozen components - Sun exposure (UV risks) can be increased with reflection from snow and at higher altitudes | 3 Likely | 3 Serious | 9 High Risk | - Training on cold stress prevention and reminders during briefings, including early warning signs of cold stress - Ensure staff dress in layers and insulated, waterproof and wind-resistant clothing, gloves, headgear and boots - Equip workers with high-vis and reflective outerwear - Schedule regular breaks and rotations in heated shelters - Provide warm, non-caffeinated drinks in break areas - Perform buddy-checks to monitor alertness and cold stress - Encourage proper rest, nutrition, and hydration before shifts - Keep work areas, platforms, and paths clear of snow/ice and apply sand, salt, grit, or traction aids to walking surfaces - Conduct frequent inspections and de-icing of equipment - Wear UV-rated sunscreen and eye protection, as applicable | 2 Possible | 3 Serious | 6 Med. Risk | See resources for employers from WorkSafeBC and go2HR on cold stress prevention Suspend or modify fleet maintenance operations when visibility and temperature fall below safe limits as part of weather monitoring and emergency plans |
| Overnight coverage and fatigue | - Negative impact to mental well-being and health - Slower reaction times or reduced ability increasing incident risks - Communication errors - Increased absenteeism | 3 Likely | 2 Moderate | 6 Med. Risk | - Schedule regular breaks during overnight shifts - Limit shift length and avoid excessive overtime - Avoid scheduling critical tasks during low-alert periods - Train staff to recognize and report signs of fatigue - Encourage healthy sleep habits and work-life balance - Share mental health resources and support options - Supervisors to check-in to assess fit for duty and alertness | 2 Possible | 2 Moderate | 4 Med. Risk | See resources for employers from WorkSafeBC and go2HR on psychological safety, mental health and fatigue |