|  | Industry/Sector: | Ski Hill and Areas | Department/Job Position: | Terrain Parks |
| --- | --- | --- | --- | --- |
|  | Common Tasks and Duties: | - Setting up, opening, inspecting, and maintaining jumps, rails, boxes, halfpipes, fencing, and signage - Shovelling, raking, and shaping snow to maintain features - Assisting and working around snowcat operations and mobile equipment - Using hand and power tools when performing maintenance or repair on park features - Setting up and adjusting features and park fencing/barriers - Monitoring guest use of the park, enforcing park rules, and providing guidance on safe use - Communicating with patrol, lift ops, and snowmaking/grooming - Responding to incidents and assisting patrol with injured guests | Reasonably Foreseeable or Common Hazards: | - Slips, trips and falls, including tree wells, snow instability, embankments and other hill hazards - Hand or eye injuries from using hand or power tools - Collisions with objects, guests, workers, equipment - Struck by/against (e.g. snowcat, UTV, snowmobile) - Working from heights - Falling objects (e.g. ice, snow, guest items) - Musculoskeletal injuries (MSI) and ergonomics - Severe weather and cold stress - Harassment and violence, working alone - Emergency situations (e.g. injuries and lost child) - Fatigue and psychological hazards |

Employers are to review this HIRA document and customize to their location, job tasks and site-specific hazards. To edit a row, simply use Microsoft word to edit (e.g. edit text, change risk ratings, delete a row, or add a new row). When determining risk follow the Guidance Document - Assessing Risk for HIRA Tools. When identifying controls, be sure to use the Hierarchy of Controls to control risk through elimination, substitution, engineering controls, administrative controls and Personal Protective Equipment (PPE). Once complete, this should be reviewed by the JHSC or a Worker H&S Rep, and reviewed annually thereafter.

|  | Full Name | Job Position | Signature | Date |
| --- | --- | --- | --- | --- |
| Completed By: |  |  |  |  |
| Reviewed By: |  |  |  |  |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Setting up and opening terrain park | - Slips, trips and falls - Tree wells, snowpack instability, embankments and hill hazards - Collisions with objects, guests, workers, or equipment - Falling ice/snow or other objects - Contact with sharp edges of rails - Unstable/unsecure park features - MSI (strains/sprains) risks - Severe weather and cold stress | 3 Likely | 3 Serious | 9 High Risk | - Recruitment, training, skills assessment, and supervision should confirm ski competencies/abilities for terrain park - Designated routes and clear communication and briefings - Proper equipment and materials, including PPE (e.g. safety headgear, gloves, eye protection and reflective materials) - Ensure functioning of radios, Sat. comms/inReach devices - Pre-use inspection/testing (gear, equipment, radios, GPS) - Communicate and flag (mark/rope off) hazards immediately - Maintain situational awareness, avoid distractions - See HIRA for control measures for MSI and severe weather | 2 Possible | 3 Serious | 6 Med. Risk | Briefings to provide overview on daily hazards, expected weather, fit for duty, planned and alternate routes, safe work practices, PPE, and rescue/emergency procedures Ensure features are properly designed and secured in accordance with engineering and manufacturer’s instructions |
| Clearing snow/ice from terrain park huts, line ups and features | - Slips, trips and falls - Falling ice/snow or other objects - Contact with sharp edges - Severe weather and cold stress - Significant MSI risks due to heavy and rigorous movement associated with snow clearing and ramp maintenance | 3 Likely | 3 Serious | 9 High Risk | - Regular de-icing and snow removal to minimize risk of excessive snow and ice build-up or equipment failure - Training on proper shovelling and snow removal techniques - Different sized shovels and ergonomic/extendable tools - Set up work areas to prevent tools/materials and ice/snow from falling and use work delineation (e.g. barriers, cones) - Proper footwear and traction aids (e.g. cleats, Yaktrax, mats) - Supervision to monitor correct techniques and controls - See HIRA for general control measures for MSI risks, cold stress and harassment/violence if working with the public | 2 Possible | 3 Serious | 6 Med. Risk | Use snowblowers only when trained and when safe to operate without creating risks to other workers or guests in the area |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Assisting snowcat operations and working around mobile equipment (e.g. excavator) | - Slips, trips and falls - Struck by or crushing hazards from moving snowcats or other mobile equipment - Struck by attachments (blades, rakes) or entanglement from winch cables - Limited operator visibility leading to “blind spots” - Noise and loud environments - Sprayed snow and accumulated snow piles - Severe weather and cold stress | 3 Likely | 3 Serious | 9 High Risk | - Establish clear work areas and exclusion zones - Only qualified and trained workers permitted in work zone - Provide training on potential blind spots and hand signals - Proper equipment and materials, including PPE (e.g. safety headgear, gloves, eye protection and high-vis clothing) - Ensure functioning of radios, Sat. comms/inReach devices - Pre-use inspections to ensure safety devices (e.g. lights, alarms, guards etc.) are operational - Schedule work during off-hours or controlled closures - Prohibit entry into areas when snowcats are operating and secure with signage and barriers to prevent entry - Approach mobile equipment after it has stopped, and only after eye contact with the operator and signal confirmation | 2 Possible | 3 Serious | 6 Med. Risk | Suspend or modify snowcat and other mobile equipment use when visibility and temperature fall below safe limits or other unsafe conditions as part of weather monitoring and emergency plans Ensure adequate lighting and visibility - if not possible, take additional precautions (e.g. spotters, closures etc.) |
| Maintaining features through using hand and power tools (e.g. drills, bearing press, carpentry) | - Cuts, punctures, or abrasions from hand or power tools - Rotating and moving parts - Eye injuries from metal shards, bearing fragments or debris - Faulty or defective equipment - Noise and vibration hazards - Electric shock from improperly maintained tools - Falls from step stools or ladders - MSI (strains/sprains) risks - Severe weather and cold stress | 3 Likely | 3 Serious | 9 High Risk | - Only qualified and trained personnel to work with equipment - Use guarding and barriers around exposed moving parts - Do not defeat or bypass safety guards or devices - Follow manufacturer’s instruction for safe use, pre-use inspection and maintenance, and safe work procedures - Ensure all power tools are double-insulated or properly grounded and rated for outdoor use - Wear PPE (e.g. gloves, safety glasses, hearing protection) - Avoid loose clothing, secure hair, jewellery, and ID badges - Maintain situational awareness, avoid distractions - Set up exclusion areas with work delineation (e.g. barriers, cones, signage etc.) to keep others out of the work areas - Store tools securely to prevent slips, trips, or drops - Set up work areas to prevent tools/materials from falling and use adequate work delineation (e.g. barriers, cones etc.) | 2 Possible | 3 Serious | 6 Med. Risk | Suspend or modify terrain parks when visibility and temperature fall below safe limits or other unsafe conditions as part of weather monitoring and emergency plans Safe ladder use includes pre-use inspection of ladders and equipment, ensuring CSA or ANSI-certified, maintaining 3-points of contact, training, ensuring proper set-up and using work delineation and spotters, see WorkSafeBC Ladder Safety |
| Performing hot work (e.g. welding, cutting, grinding, torch use) | - Burns from sparks, molten metal, or hot surfaces - Fire or explosion from sparks contacting combustible materials - Eye injuries from arc flash, flying disc, sparks, or debris - Respiratory hazards from welding fumes, grinding dust, or smoke - Noise and vibration hazards | 3 Likely | 3 Serious | 9 High Risk | - Only trained and qualified personnel to perform hot work - Hot work permit system in accordance with WorkSafeBC - Remove or shield combustible and flammable materials - Use required PPE (e.g. face shield, safety glasses, gloves, FR clothing, hearing protection, respirator etc.) - Ensure fire extinguishers are available with workers trained - Assign a fire watch during and after hot work - Use local exhaust ventilation or portable fume extractors - Restrict access to hot work areas using work delineation | 2 Possible | 3 Serious | 6 Med. Risk | If contracting work with increased risk, ensure contractor safety is managed through controls such as pre-qualification, confirmation of valid insurance (including WSBC clearance letter), hazard assessments, verification of qualifications and training, and safe work procedures and equipment |
| Monitoring guest use of the park, enforcing park rules, and providing guidance on safe use | - Slips, trips and falls - Collisions with objects, guests, workers, or equipment - Unpredictable behaviour - Busy/congested areas - Falling ice/snow or other objects - Working alone and in isolation - MSI (strains/sprains) risks - Severe weather and cold stress - Harassment and violence - Psychological safety and fatigue | 2 Possible | 3 Serious | 6 Med. Risk | - Recruitment, training, skills assessment, and supervision should confirm ski competencies/abilities for terrain park - Clear instruction, signage and communication - Position workers in safe zones to avoid collision - Designated areas/routes, enforce spacing and speed control - Proper equipment and materials, including PPE (e.g. safety headgear, gloves, eye protection and high-vis clothing) - Ensure functioning of radios, Sat. comms/inReach devices - Pre-use inspection/testing (gear, equipment, radios, GPS) - Checklist to confirm necessary supplies (e.g. water, energy snacks/food, first aid kit, markers) and adequate breaks - Communicate and flag (mark/rope off) hazards immediately - Maintain situational awareness, avoid distractions - See HIRA for general control measures for MSI risks, cold stress and harassment/violence if working with the public | 2 Possible | 2 Moderate | 4 Med. Risk | As necessary, perform work after hours or close features and terrain parks to prevent use during inspections and maintenance Assign terrain park duties based on skiing/boarding ability and qualifications, with stronger instructors covering more technical and advanced terrain Ensure adequate staffing, with extra staff during peak times |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Harassment and violence due to working with the public | - Verbal abuse or aggression during delays, lineups or first aid - Physical contact or assault such as pushing, grabbing, or striking - Sexual harassment including inappropriate comments, gestures, or contact - Stress and fatigue from repeated or escalating incidents | 2 Possible | 3 Serious | 6 Med. Risk | - Harassment and violence prevention program in accordance with WorkSafeBC requirements - Working Alone Policy with established check-in procedures - Training and awareness on harassment, violence, conflict resolution, de-escalation, and recognizing warning signs - Reporting procedures for response and incident reports - Communication devices (e.g. radios) for immediate contact with supervisors or additional patrol - Increased security or patrol presence during peak periods - Support resources such as post-incident debriefing, mental health supports, or employee assistance programs | 2 Possible | 2 Moderate | 4 Med. Risk | See resources for employers from WorkSafeBC and go2HR on workplace harassment and violence prevention Avoid work with increased risk when working alone. When possible, have staff work in pairs or groups, particularly in locations and with tasks that are increased risk |
| Severe weather and cold stress | - Cold stress injuries such as frostbite and hypothermia - Impaired decision-making and slower reaction times - Slips, trips, and falls from snow and ice accumulation on surfaces - Reduced visibility from blowing snow, fog, or whiteout conditions - Equipment malfunctions or failures from frozen components - Sun exposure (UV risks) can be increased with reflection from snow and at higher altitudes | 3 Likely | 3 Serious | 9 High Risk | - Training on cold stress prevention and reminders during briefings, including early warning signs of cold stress - Ensure staff dress in layers and insulated, waterproof and wind-resistant clothing, gloves, headgear and boots - Equip workers with high-vis and reflective outerwear - Schedule regular breaks and rotations in heated shelters - Provide warm, non-caffeinated drinks in break areas - Perform buddy-checks to monitor alertness and cold stress - Encourage proper rest, nutrition, and hydration before shifts - Keep work areas, platforms, and paths clear of snow/ice and apply sand, salt, grit, or traction aids to walking surfaces - Conduct frequent inspections and de-icing of equipment - Wear UV-rated sunscreen and eye protection, as applicable | 2 Possible | 3 Serious | 6 Med. Risk | See resources for employers from WorkSafeBC and go2HR on cold stress prevention Suspend or modify terrain parks when visibility and temperature fall below safe limits or other unsafe conditions as part of weather monitoring and emergency plans |
| Musculoskeletal injuries (MSI) and ergonomics | - Repetitive motions from shovelling, raking, and shaping features or handling tools - Awkward postures when bending, kneeling, or reaching to set rails, boxes, or signage - Overexertion when lifting or moving heavy/awkward items (rails, boxes, fencing, or blocks) - Pushing or pulling when relocating fencing, dragging rails, or moving sleds with equipment - Carrying loads such as tools, signage, or gear for extended distances across uneven terrain | 3 Likely | 3 Serious | 9 High Risk | - Training on MSI prevention and proper techniques - Reminders during pre-shift briefings on MSI prevention - Rotate tasks (shovelling/material handling) throughout shift - Warm-up, stretch and take breaks including micro-breaks - Use proper lifting techniques or mechanical aids - Different sized shovels and ergonomic/extendable tools - Bend using half-kneel, instead of crouching or back bending - Break heavy loads (e.g. equipment) into smaller loads - Use team lifts for heavy/awkward items (transport gear) - Use equipment with wheels, sliders, or handles, if possible - Push carts instead of pulling to reduce risk of MSIs - Keep pathways clear to reduce resistance and difficulty - Report and replace worn or broken equipment - Store heavier items at waist level to avoid unnecessary lifting - Encourage early reporting of discomfort or MSI symptoms | 2 Possible | 3 Serious | 6 Med. Risk | See resources for employers from WorkSafeBC and go2HR on MSI prevention Encourage maintenance of physical fitness during off-season and throughout season When possible, have snow mobiles or mobile equipment move features, equipment or other heavy/awkward loads, operators must be trained and work in accordance with WorkSafeBC Mobile Equipment requirements |
| Responding to falls, injuries and emergencies | - Worker/guest medical emergency - Slips, trips and falls - Malfunction/failure of systems and equipment - Contact with blood or other bodily fluids (biological hazards) - Severe weather and cold stress - Struck by/against from collisions, or ski equipment - Psychological stress responding to emergencies - Disruption to business operations | 3 Likely | 3 Serious | 9 High Risk | - Ensure first aid assessment and medical program in place - Only trained and qualified personnel to perform emergency and rescue operations with emergency equipment and PPE - Tabletop exercises and drills to reinforce training - Scene assessment (e.g. unstable structures/snow, traffic, falls, electrical risks etc.) to confirm safe for response - Wear PPE (e.g. gloves) and use universal precautions for infection control and dispose of waste appropriately - Provisions for debriefing and psychological safety and mental health supports for workers involved in or witnessed critical/traumatic incidents - Business continuity with crisis communication plans in place | 2 Possible | 3 Serious | 6 Med. Risk | Ensure first aid requirements are in place in accordance with WorkSafeBC (including drills), and system in place to ensure certifications and equipment (e.g. kits and AEDs) are up to date and maintained Carry emergency kit and gear (e.g. first aid kit, lighting, blanket, survival pack, tow rope, extra fuel), ensure regularly inspected and complete |