|  | Industry/Sector: | Restaurants and Dining Establishments | Department/Job Position: | Breweries (Tours, Tasting Room, Production) |
| --- | --- | --- | --- | --- |
|  | Common Tasks and Duties: | - Set up and maintain service and production areas, including glassware, taps, and service stations - Prepare and serve beer for tastings and sales - Operate brewery equipment as trained e.g. pumps, hoses, keg washers, dishwashers, and canning/bottling/kegging equipment - Handle and transport kegs, cases, grain, hops, and supplies - Assist with cleaning and sanitizing tanks, lines, and surfaces - Interact with guests and customers, support tours and tastings - Work around brewing equipment, confined or enclosed spaces, ladders, mobile equipment, and production hazards - Manage waste, recycling, and cleanup of spills or broken glass | Reasonably Foreseeable or Common Hazards: | - Musculoskeletal injuries (MSI) and ergonomics - Working with the public, violence and harassment - Slips, trips and falls - Struck by/against incidents, mobile equipment - Confined spaces and pressurized systems - Carbon dioxide (CO2) hazards - Burns from boiling liquids, steam or hot surfaces - Small cuts and lacerations, broken glass - Conveyors and equipment for cans, bottles and kegs - Noise and loud environments - Fatigue and psychological hazards |

Employers are to review this HIRA document and customize to their location, job tasks and site-specific hazards. To edit a row, simply use Microsoft word to edit (e.g. edit text, change risk ratings, delete a row, or add a new row). When determining risk follow the Guidance Document - Assessing Risk for HIRA Tools. When identifying controls, be sure to use the Hierarchy of Controls to control risk through elimination, substitution, engineering controls, administrative controls and Personal Protective Equipment (PPE). Once complete, this should be reviewed by the JHSC or a Worker H&S Rep, and reviewed annually thereafter.

|  | Full Name | Job Position | Signature | Date |
| --- | --- | --- | --- | --- |
| Completed By: |  |  |  |  |
| Reviewed By: |  |  |  |  |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Interacting with customers and providing tours, sales and tasting experiences | - MSI risks e.g. repetitive motion, overexertion, reaching, awkward postures (see more below) - Aggressive guests, violence and harassment with increased risk from alcohol consumption | 3 Likely | 3 Serious | 9 High Risk | - Violence/harassment policies, risk assessments and training - Security measures based on risk as part of violence risk assessment e.g. CCTV, strategic layout (e.g. visible, counters, unobstructed egress, lighting), cash handling protocols etc. - Ensure alcohol compliance and Serving It Right standards - Clear conditions of entry signage posted and communicated - Brewery staff trained in de-escalation and refusal techniques - Maintain situational awareness, avoid distractions and rushing - Working Alone Policy with established check-in procedures - Devices for immediately summoning assistance e.g. radios, apps, panic button, whistles etc. | 2 Possible | 3 Serious | 6 Med. Risk | See resources for employers from WorkSafeBC and go2HR on de-escalation scenarios and Safety Talk on navigating challenging interactions and other workplace violence prevention resources Avoid work with increased risk when working alone. When possible, have staff work in pairs or groups, particularly in locations or at times with increased risk |
| Working within the brewery, tasting room and production areas | - MSI risks e.g. repetitive motion, overexertion, reaching, awkward postures (see more below) - Slips, trips and falls - CO2 exposure (see more below) - Equipment, conveyors, and moving parts (see more below) - Noise and loud environments - Low lighting, poor visibility | 3 Likely | 2 Moderate | 6 Med. Risk | - Inspect the area, remove objects that pose tripping hazards - Wear slip-resistant footwear, ensure adequate floor traction - Clean up spills immediately, and use “Wet Floor” signage - Secure hoses, cords and cables to prevent tripping - Use step stools to reach higher areas and shelves, do not stand on counters, tables, shelves, chairs or other objects - Use rubber matting and dams where spills often occur - Use adequate lighting in tasting room and production areas - Wear hearing protection and implement hearing conservation program based on WorkSafeBC requirements | 2 Possible | 2 Moderate | 4 Med. Risk | Other tasks within this HIRA identify more specific risks and associated controls for work with increased risk e.g. broken glass, boiling liquids, steam and hot surfaces, mobile equipment (e.g. forklifts), equipment (e.g. conveyors, canners/bottlers, hoses etc.), working at heights, and chemical products including CO2 |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Handling and using canners/ bottlers/keggers, tools or other sharp objects | - Broken glass and bottles - Sharp glass, can edges, and objects causing cuts and lacerations - Tools, equipment, and moving objects or blades - Improper use and storage | 3 Likely | 2 Moderate | 6 Med. Risk | - Where possible, minimize handling of glassware and bottles - Safe work procedures including training, disposal container, equipment and PPE for broken glassware and bottles - Install glass racks and storage systems to prevent breakage - Regular inspection of cans/glass for edges, chips or cracks - Dispose of glass and metal in clearly marked containers - Protocols and training for cleaning and canning/bottling - Use the right tool for the job, consider safety utility knives - Use PPE e.g. eye protection, cut-resistant gloves etc. | 2 Possible | 2 Moderate | 4 Med. Risk | See WorkSafeBC Culinary videos for working safely in the kitchen: https://www.worksafebc.com/ en/resources/health-safety/videos/kitchen-safety-focusing-on-safety |
| Working with hot and boiling liquids, steam and surfaces e.g. hot tanks | - Exposure to hot liquids or surfaces causing burns - Steam from glass and dishwasher - Hot surfaces from tanks and equipment - Eye injuries due to splashes - MSI risks (see more below) | 3 Likely | 3 Serious | 9 High Risk | - Keep hot liquid use or sources of steam (e.g. piping, tanks) areas clearly designated and away from high-traffic zones - Ensure hot water taps and hoses are properly maintained, free of leaks with gaskets and connections in good condition - Do not open steam pipes without locking out and purging - Install boil-over protection systems in brew kettle, if possible - Consider adding insulation and signage to hot surfaces - Ensure PPE (e.g. apron, face protection, gloves etc.) are used and long-sleeved shirts and pants are worn | 2 Possible | 3 Serious | 6 Med. Risk | Provide training for workers on hazards and controls, and identify hot surfaces and sources If using insulation, ensure it is appropriate (e.g. foodsafe and chemical resistant) for the environment |
| Pressurized systems e.g. pumps, hoses for cleaning and production | - Slips, trips and falls - High-pressure fluid, gas, or air release causing injection injuries - Hose or fitting rupture, failure or malfunction releasing energy - Eye injuries from flying debris or materials - Crush or pinch point hazards - MSI (strains/sprains) risks - Noise and vibration hazards | 3 Likely | 3 Serious | 9 High Risk | - Only qualified and trained personnel to work with pressurized systems and equipment, put away safely when not in use - Follow manufacturer’s instruction for safe use, pre-use inspection and maintenance, and safe work procedures - De-energize and Lockout/Tagout (LOTO) equipment before performing cleaning or maintenance - Use manufacturer-specified tools, fittings, and rated hoses to prevent leaks and failures, do not substitute with other parts - Inspect hoses, fittings, and cylinders regularly for wear, cracks, or damage, and replace immediately - Wear PPE e.g. gloves, eye protection, hearing protection, safety footwear etc. | 2 Possible | 3 Serious | 6 Med. Risk | Report and repair leaks immediately, do not operate pressurized systems or any equipment with known defects or safety concerns Secure hoses and cords to prevent damage and trip hazards, ensure proper hose length and secure in designated areas when not in use |
| Working in or around confined and enclosed spaces e.g. grain bins and silos, fermenters, process tanks, brewery equipment etc. | - Asphyxiation (e.g. oxygen deficiency or toxic gas exposure) - Limited entry/exit complicating emergency response - Entrapment or moving parts - Building-related hazardous materials or biological agents - MSI risks (see more below) | 2 Possible | 4 Critical | 8 Med. Risk | - Cleaning-In-Place (CIP) to avoid entering confined spaces - Confined spaces are inventoried, assessed, and labelled - Only qualified and trained personnel to enter confined spaces - Use a confined space permit and entry checklist - Atmosphere tested for oxygen and toxic gasses - Wear required PPE, respirators require fit testing - Confined space entry procedures oxygen and gas detection - Ventilate the space before and during entry - Isolate energy sources, Lockout/Tagout (LOTO) before entry - Standby person with communication device and rescue plan | 2 Possible | 3 Serious | 6 Med. Risk | See WorkSafeBC information on confined spaces specific to breweries: https://www.worksafebc.com/ en/resources/health-safety/risk-advisory/confined-spaces-in-craft-brewing |
| Using equipment e.g. conveyors, canners/bottlers/ keggers, hoses, crushers, press etc. | - MSI risks (see more below) - Slips, trips and falls - Struck by/against incidents - materials - Crush or pinch point hazards - Entanglement or moving parts - Electrical shock and hazards - Faulty or defective tools or equipment - Noise and vibration hazards | 3 Likely | 3 Serious | 9 High Risk | - Provide workers with training and instruction on equipment safety, including safe use and start-up/shut-down procedures - Ensure no loose clothing or jewellery, hair tied back to reduce risk of entanglement or caught in hazards - Ensure machine guards in place and accessible emergency stops, ensure visible and train workers in function and use - Do not defeat or bypass safety guards or devices - Encourage staff to report faulty or broken equipment - Use CSA-approved electrical equipment and GFCI protection - Keep electrical connections away from water and spills - Do not overload circuits or power bars, maintain clearances | 2 Possible | 3 Serious | 6 Med. Risk | Reminders on electrical safety: not overloading outlets, unplugging by the plug (do not pull cord), reporting damaged equipment (including cords and plugs), and always unplugging equipment before cleaning (e.g. kettles, lighting etc.) De-energize and Lockout/Tagout (LOTO) equipment before performing cleaning or maintenance |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Using hazardous products e.g. sanitizers, cleaners, caustics, acids, flammables, and compressed gases | - Compressed CO2 and gas tanks e.g. tip over, asphyxiation, frostbite, projectile hazards etc. - Exposure to chemical hazards e.g. chemical burns, splashes to eyes, respiratory or skin irritation - Unlabelled or illegible containers or gas cylinders - Spills and environment concerns | 2 Possible | 3 Serious | 6 Med. Risk | - WHMIS training for staff using hazardous products - Supplier and workplace labels in place and legible - Safety Data Sheets (SDS) referenced and available - Safe handling practices and PPE as required by label/SDS - Employer to assess products for hazards before procuring - Ensure adequate and good ventilation in working areas - First aid on site with emergency eyewash - Compressed gas cylinders (e.g. CO2 tanks) to only be used by trained workers in accordance with SDS in ventilated areas - Ensure proper storage and securement against tipping - Install CO2 monitoring alarms and train workers on what to do if CO2 high alarms are activated - Take necessary precautions for use and storage of flammables including when distilling or decanting alcohol | 2 Possible | 2 Moderate | 4 Med. Risk | Educate workers on hazards of WHMIS and consumer products (and associated symbols) e.g. Windex or other products that may not be WHMIS-controlled Use gas cylinders with the required regulators, hoses (check hoses for leaks and in good condition), fittings and PPE e.g. gloves, eye protection Ensure applicable emergency and exposure control plans are in place for CO2 related hazards |
| Operating and working around vehicles/trucks, including use of mobile equipment e.g. forklifts, pallet jacks etc. | - Slips, trips, and falls - Collisions with workers, tour groups, or equipment - Struck by moving vehicles or equipment - Traffic management hazards - Working alone and in isolation - Noise and vibration hazards - Working overnight and fatigue | 3 Likely | 4 Critical | 12 High Risk | - Operator training and competency for operating vehicles and mobile equipment, confirmation of license and certification - Separate pedestrian and vehicle traffic, where possible - Clearly marked touring routes and restricted access areas - Workers and visitors to wear PPE and high vis, as applicable - Conduct pre-use inspections of vehicles and equipment - Load carrying and tow restrictions, ensure loads are secured before travel and use of lights, mirrors, horns as applicable - Prohibit standing between vehicles and docks - Use spotters in areas with pedestrian traffic and congestion - Maintain situational awareness, use horns at intersections - Follow speed limits, wear seatbelts, and prohibit cellphones - Park equipment safely when not in use, remove keys and use wheel chocks as applicable - Drivers and operators to follow site safety rules, fit for duty with clear signage and instructions identifying requirements | 2 Possible | 3 Serious | 6 Med. Risk | See WorkSafeBC information for forklift and material handling equipment: https://www.work safebc.com/en/health-safety/tools-machinery-equipment/cranes-mobile-equipment/types/forklifts-materials-handling-equipment |
| Working at heights e.g. step stools, ladders and Elevating Work Platforms (EWPs) | - Slips or trips on uneven or slippery surfaces - Falling from heights - Falling tools or materials posing risk to others below - Overreaching or loss of balance while working at heights - Faulty or defective equipment or ladders - Distractions or unsafe use of ladders or EWPs | 3 Likely | 4 Critical | 12 High Risk | - Utilize the fall protection hierarchy (e.g. elimination, guardrails, fall/travel restraint and fall arrest) to eliminate or guard against fall hazards as the preferred methods - Only qualified and trained personnel to work from heights - Follow safe work procedures and wear required PPE - Assess the area for overhead hazards (e.g. fans or electrical equipment), maintain limits of approach - Conduct pre-use inspections of ladders and EWPs/lifts - Maintain three points of contact when climbing and working - Set up work areas to prevent tools/materials from falling and use adequate work delineation e.g. barriers, cones etc. - Use spotters or barricades to protect others below - Follow manufacturer’s instructions and WorkSafeBC requirements for ladder inspection, safe use, and storage - Follow manufacturer’s instructions and WorkSafeBC requirements for inspection, safe use and storage of EWPs e.g. scissor lifts or aerial lifts | 2 Possible | 3 Serious | 6 Med. Risk | WorkSafeBC recommends adding dry ingredients to tanks using pneumatic conveyance, recirculation systems, platform ladders, or scissor lifts Safe ladder use includes pre-use inspection of ladders and equipment, ensuring CSA or ANSI-certified, maintaining 3-points of contact, training, ensuring proper set-up and using work delineation and spotters See WorkSafeBC information on ladder safety: https://www.work safebc.com/en/health-safety/tools-machinery-equipment/ladders |

| Task | Hazards/Risks | Inherent (Before Controls) Risk |  |  | General Control Measures | Residual (After Controls) Risk |  |  | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Probability | Severity | Rating |  | Probability | Severity | Rating |  |
| Storage racks, loading and unloading and general storage | - Overloading leading to tipping or unsecured items falling - Poor maintenance, resulting in deterioration or damage - Unsafe loading and unloading - Rack collapse or failure due to impact, damage, or overloading - Blocked aisles, exits, or fire equipment | 3 Likely | 4 Critical | 12 High Risk | - Post rated load capacities on all racking, do not exceed - Install, modify, and repair racks per manufacturer or engineer specifications and only by a qualified person - Conduct inspections for damage, missing parts, or instability - Take damaged racks out of service, and have a qualified person repair per manufacturer/engineer specifications - Train workers on safe stacking, inspection, and procedures - Keep aisles, exits, and fire equipment clear of storage items - Maintain required clearance(s) between racking for fire separation, evacuation and away from fire/life safety devices | 2 Possible | 3 Serious | 6 Med. Risk | See WorkSafeBC information on storage rack and racking requirements: https://www.worksafebc.com/ en/health-safety/tools-machinery-equipment/storage-racks |
| Storing materials in walk-in fridges and cooler rooms | - Cold stress illnesses or injuries - Ice build-up and condensation - Slips, trips and falls - Power outage, limited visibility - Locked inside cooler - Malfunctioning door latches or internal release mechanism | 2 Possible | 3 Serious | 6 Med. Risk | - Ensure workers dress in layers for work conditions - Provide training for workers on safe use and inspection of walk-in fridges and coolers, including emergency lighting - Slip resistant flooring/mats, do not store items on the floor - Inspect doors, latches, alarms, and release mechanisms - Implement controls e.g. limited time, buddy-system, PPE etc. - Working Alone Policy with established check-in procedures | 2 Possible | 2 Moderate | 4 Med. Risk | Walk-in fridges and freezer doors must always be openable from the inside and equipped with clearly visible internal release mechanisms and lighting and/or alarm systems to ensure workers can safely exit in an emergency |
| Musculoskeletal injuries (MSI) and ergonomics | - Repetitive motions from pouring, lifting glassware, handling hoses, and cleaning tasks - Awkward postures when reaching into coolers, tanks, racking, or under counters - Overexertion from lifting or moving kegs, cases, grain bags, hoses, and barrels - Pushing and pulling carts, pallet jacks, mobile bins, or equipment - Prolonged standing, bending, and twisting during production, packaging, or busy service times | 3 Likely | 3 Serious | 9 High Risk | - Training on MSI prevention including safe lifting and hose handling and body positioning techniques - Reminders during pre-shift briefings on MSI prevention - Warm-up, stretch and take breaks, including micro-breaks - Use safe lifting techniques, lift in pairs or use a cart or dolly when moving heavy materials e.g. kegs, cases, bottles etc. - Push carts and dollies instead of pulling to reduce risk of strain - Inspect and maintain equipment regularly including wheels - Rotate tasks throughout the shift to reduce repetitive motions - Report and replace worn or broken equipment - Position frequently used and heavier items in close proximity and at waist height to avoid unnecessary lifting and bending - Consider anti-fatigue mats for prolonged standing - Encourage early reporting of MSI signs and symptoms | 2 Possible | 3 Serious | 6 Med. Risk | See resources for employers from WorkSafeBC and go2HR on MSI prevention and ergonomics |
| Late working hours and fatigue | - Mental well-being and health - Slower reaction times - Communication errors - Increased absenteeism - Safety and service quality affected | 3 Likely | 2 Moderate | 6 Med. Risk | - Schedule regular breaks, and ensure adequate staffing - Limit shift length and avoid excessive overtime - Train staff to recognize signs of fatigue and a supportive culture that prioritizes rest and wellness - Encourage healthy sleep habits and work-life balance - Maintain situational awareness, avoid distractions and rushing | 2 Possible | 2 Moderate | 4 Med. Risk | See resources for employers from WorkSafeBC and go2HR on psychological safety, mental health and fatigue |

Note: This Hazard Identification and Risk Assessment (HIRA) tool provides a general overview of common hazards and controls associated with tourism and hospitality operations. Manufacturing and warehousing involved in the production of beer (alcohol or non-alcohol) and other products must also comply with applicable WorkSafeBC requirements. For additional guidance on brewery-specific hazards, controls, and regulatory requirements, refer to resources provided by WorkSafeBC.

- WorkSafeBC: https://www.worksafebc.com/en/resources/health-safety/books-guides/health-safety-craft-breweries-distilleries-bk164

- WorkSafeBC: https://www.worksafebc.com/en/resources/health-safety/videos/health-and-safety-in-craft-brewing