### Lockout Tagout (LOTO)

### Purpose

This Safe Work Procedure explains how to safely de-energize and Lockout Tagout (LOTO)

###### This procedure supports employer and worker duties under the Workers Compensation Act.

### Know Your Responsibilities

##### Employers Must

- Provide a safe workplace and equipment

- Identify, assess, and control hazards

- Train and supervise workers in safe practices

- Maintain SWPs, emergency procedures, and safety policies

- Investigate and report incidents

##### Workers Must

- Follow safe work practices and procedures

- Wear and use required PPE

- Report hazards or unsafe conditions

- Participate in training and drills

- Work safely to protect self and others

### What Could Go Wrong?

- Unexpected start-up of machines, appliances, or equipment

- Release of stored energy from electrical, mechanical, or pressurized systems

- Injury from moving parts, hot surfaces, or chemicals

- Confined spaces, panel-controlled machines, or multiple energy sources creating additional hazards

### Before You Start

- Be trained and authorized

- Take time to assess the situation and plan your approach

- Review manufacturer instructions and energy-isolating devices for the specific equipment

Special Considerations

- Machines with multiple power sources require each source to be isolated and locked individually

- Plug-connected equipment should be unplugged and the plug kept in sight of the worker

- Panel-controlled machines require correct breaker identification, locking of the panel, and application of a personal lock

### Required PPE

- Gloves suitable for electrical or mechanical hazards

- Apron or protective clothing

- Safety glasses or goggles

### Safe Work Steps

- Identify the equipment, machine, or system that needs to be locked out

- Inform all affected workers and departments that the equipment will be shut down

- Confirm that shutting down the equipment will not create a hazard for others

- Power down the equipment following standard operating procedures

- Identify all potential energy sources, including electrical, mechanical, hydraulic, pneumatic, or chemical

- Use the appropriate energy-isolating device(s) to disconnect or block power at each source

- Apply your personal lock and tag that includes your name, date, and equipment being locked out

- Use a group lockout device if multiple workers are performing tasks; each worker applies their own lock and tag

- Confirm all workers are clear of the equipment and test to ensure energy is properly isolated and the system is non-operational

- Perform the cleaning, maintenance, inspection, or repair task according to safe work procedures and manufacturer instructions

- Remove all tools, debris, and replace any guards before removing locks

- Remove your own lock only; never remove another worker’s lock

- If a worker who applied a lock is unavailable, the supervisor follows protocol to safely remove the lock and ensures the worker is notified

- Once all locks are removed and workers are clear, restore power and test equipment before resuming full operation

### In Case of Emergency

- Report hazards immediately to your supervisor, Joint Health and Safety Committee, or Worker Health and Safety Representative (if applicable)

- Get first aid immediately for cuts or injuries.

- Report all incidents, near misses, and injuries.

### Additional Safety Resources

- WorkSafeBC Lockout https://www.worksafebc.com/en/health-safety/tools-machinery-equipment/lockout

- Canadian Centre of Occupational Health and Safety (CCOHS) Lockout https://www.ccohs.ca/oshanswers/hsprograms/lockout.html

- Canadian standard CSA Z460-20 Control of Hazardous Energy - Lockout and Other Methods

Date of Release:      
Date of Review:                                                    Reviewed by: